- SOUTHERN NATURAL GAS COMPANY

- DESCRIPTION OF DESIGN AND OPERATION

- OF PIPELINE SYSTEM

Southern operates an interstate pipeline system for the transportation of natural gas in the states of Texas, Louisiana, Mississippi, Alabama, Georgia, South Carolina, Florida and Tennessee. Southern also holds a blanket certificate of public convenience and necessity pursuant to Section 284.281 et seq. of the Federal Energy Regulatory Commission’s Regulations under the Natural Gas Act authorizing it to make unbundled sales of natural gas.  Although such sales can be for any purpose, sales of gas by Southern have been almost exclusively limited to those to resolve shipper imbalances and sales made in conjunction with the operation of Southern’s pipeline system.

The delivery capacity of Southern’s system is currently approximately 3.7 Bcf per day based on designed system flow of gas from west to east.  Since Southern’s last rate case, in Docket No. RP04-523, Southern has increased the delivery capacity of its system through various expansions by a total of approximately 0.3 Bcf, from 3.4 to 3.7 Bcf.  These expansions were made primarily to serve new electric power generation customers on Southern’s system.  The design of these expansions followed the parameters noted below for constructing capacity to meet firm service commitments.

The western termini of Southern’s main transmission system are at Perryville, Louisiana, and Gwinville, Mississippi. Southern’s two main transmission lines (known respectively as the North Line and the South Line) used in delivering gas to its customers extend east from these two points. Supply transmission lines extend west from Perryville and south from Gwinville. As of January 31, 2009, Southern’s pipeline system, including the main transmission system, market area system, and supply area system, consisted of 7,600 miles of pipeline with 39 compressor stations having a total of 543,400 installed brake horsepower.  These statistics include Southern’s Muldon Storage Field.

Southern’s principal offshore pipeline networks are the Main Pass, South Pass, West Delta, and Breton Sound supply systems.  Southern’s offshore pipeline network is interconnected with several other pipeline networks.  Southern also has interconnections onshore with several other interstate pipeline companies, primarily in Louisiana and Mississippi.  Southern receives gas supplies at these onshore and offshore interconnections that are produced in areas remote from Southern’s pipeline system and transported by the other pipelines to these points of interconnection.

In May, 1971, Southern applied to the Federal Power Commission for a certificate of public convenience and necessity, in Docket No. CP71-273, to construct, develop, and operate the Muldon Field in Monroe County, Mississippi, as an underground storage field. The requested certificate was issued by the Commission and the project was completed and in operation early in 1972. In 1975, the Commission approved an expansion of the Muldon Storage Field. This additional construction was completed in the 1976-77 winter. On March 21, 1978, the Commission approved additional construction at Muldon. This construction was completed and the facilities placed in operation as of February 28, 1979. In Southern’s Order No. 636 restructuring proceeding, 21 Bcf of working storage gas at Muldon was reclassified to base storage gas (working storage was reduced from 52 Bcf to 31 Bcf).  When designing pipeline facilities on Southern’s system, the delivery of gas from the Muldon Storage Field is considered to be a general source of gas supply.  The Muldon Storage Field currently has a maximum deliverability of 750,000 Mcf per day.  It is one of two storage fields Southern uses in performing the storage services under Southern’s CSS Rate Schedule.

The other storage field utilized by Southern in providing its CSS storage services is the Bear Creek Field in Bienville Parish, Louisiana.  On April 3, 1979, the Commission issued a certificate to Bear Creek Storage Company authorizing the construction and operation of a gas storage field in the Pettit Reservoir, Bienville Parish, Louisiana. The Bear Creek Storage field was placed in service on July 25, 1981. Under Bear Creek’s tariff, Southern, on behalf of its customers, has the right to store and have redelivered up to one-half the capacity of the Pettit Reservoir (less base storage gas).  Deliveries from the Bear Creek Storage Field, like those from the Muldon Storage Field, are considered to be a general source of gas supply when designing pipeline facilities on Southern’s system.  Under Bear Creek’s tariff, Southern’s maximum deliverability from the Bear Creek Storage Field is 450,000 Mcf per day.

Within the two storage fields, Southern has a total working storage capacity of 60 Bcf (29 Bcf at Bear Creek and 31 Bcf at Muldon).  8 Bcf of the working storage capacity is retained and used by Southern to balance Southern’s system and maintain operational flexibility and integrity.  The remaining 52 Bcf is subscribed by Southern’s firm storage customers.

When gas storage and related transportation services are not required by Southern’s storage customers, the facilities provide increased operational flexibility and greater efficiency and reliability of service on Southern’s pipeline system. Such facilities do not affect the firm delivery capacity of Southern’s system as previously described since the facilities are dedicated for use in transportation of storage volumes whenever storage services are requested by Southern’s storage customers.

On December 1, 2001, Southern LNG’s Elba Island LNG terminal, located on the eastern end of Southern’s system, was reactivated.  The terminal was expanded in February 2006.  Elba has a total storage capacity of 7.3 Bcf and a design sendout capacity of 933,000 Mcf per day.  When deliveries from Elba are being made into Southern, the nominal delivery capacity of Southern’s system increases.  This is a function of additional gas supply flowing from east to west and consequently, for system design purposes, Elba is not currently considered to increase the capacity of Southern’s system.

In general, Southern’s main transmission system is designed and operated to deliver the coincidental maximum firm delivery obligations on a peak day, assuming hourly load patterns which vary by type of customer.  The design of Southern’s system recognizes that hourly requirements during peak periods of the day, particularly in the heating season, exceed average hourly requirements. Southern’s FERC Gas Tariff also recognizes such requirements but specifically states that Southern shall not be obligated to deliver firm transportation on any day at any point of delivery at hourly rates in excess of 6% of the firm contract quantity at that point. The delivery of peak hour requirements in excess of uniform hourly requirements is provided in part from pipeline inventory and in part from flexibility built into the pipeline system.

Pursuant to the Natural Gas Pipeline Safety Act of 1968, as amended by the Pipeline Safety Improvement Act of 2002, Southern inspects, tests, operates, replaces, and maintains its pipeline facilities in accordance with the requirements of the Department of Transportation, the Office of Pipeline Safety, and any other Federal safety standards applicable to its pipeline facilities.

- Appendix A describes the flow formula used in pipeline design.

- DESIGN DATA

- Assumptions, bases, formulae, and methods

Bases:

Temperature base		520 degrees R

Pressure base		14.73 psia

Specific Gravity		0.577-0.590 (depending on gas supply source)

Pipeline Efficiency is an empirically determined number based on historical data comparing actual performance with theoretical performance.

- Pipeline Roughness is assumed to be 0.000500 inches.

Formulae:

For pipeline design, Southern utilizes a copyrighted computer program, WINFLOW. This computer program simulates the operation of the Southern Natural Gas system. Various mass, momentum, and energy balance equations and methods as described in the WINFLOW Help Manual are used by this program to calculate pressures and horsepower along the Southern Natural Gas system. A description of the equations and variables that approximate the results of WINFLOW’s equations is set forth below:

Q=(38.77/10^6)*(Tb/Pb)*(((P1^2-P2^2)-((.0375*G(h2-h1)*Pavg^2)/(Zavg*Tavg)))/(G*Tavg*L*Zavg*f)^.5*D^2.5

Where:

Q		=	Volume of gas flowing in millions of standard cubic feet per day

P1	=	Upstream pressure, psia

P2 	=	Downstream pressure, psia

Pavg 	=	Average pressure between P1 and P2, psia

D		=	Inside pipe diameter, inches

L		=	Length of pipe, miles

Pb	=	Pressure Base, 14.73 psia

Tb	=	Temperature Base, 520 degrees Rankine

Tavg	=	Average flowing temperature, degrees Rankine

G		=	Specific gravity of gas (Air = 1.0)

Zavg	=	Average gas compressibility

f		=	Friction factor (from Colebrook – White equation)

h1	=	Upstream elevation, feet

h2	=	Downstream elevation, feet

1/(f^.5)=-4*log((e/D)/3.7+1.255/(Nre*(f^.5)))

Where:

e		=	pipe roughness, inches

Nre	=	477.5*((Q*G)/(u*D))*(Pb/Tb)

u		=	Gas viscosity.  8.05*10^(-6)lb/ft-sec

HP	=	((3.03*Q*Pb*Ts*Zs*K)/(Tb*E*(K-1)))*((r^((K-1)/K))-1)

Where:

HP	=	Compressor horsepower

r		=	Compression ratio, Pd/Ps

Ps	=	Suction pressure, psia

Pd	=	Discharge pressure, psia

Ts	=	Suction temperature, degrees Rankine

K		=	Ratio of specific heat, Cp/Cv=1.28

E		=	Overall Compressor efficiency

Zs	=	Compressibility factor at compressor suction pressure and temperature