UNITED STATES OF AMERICA

BEFORE THE

FEDERAL ENERGY REGULATORY COMMISSION

EL PASO NATURAL GAS COMPANY 	)	DOCKET NO. RP05-___

PREPARED DIRECT TESTIMONY

OF

ANDERS T. JOHNSON

Qualifications

Q.	Please state your name and business address.

A.	My name is Anders T. Johnson.  My business address is 2 North Nevada Avenue, Colorado Springs, CO 80903.

Q.	By whom are you employed and in what capacity?

A.	I am Director of the Facility Planning Department for the El Paso Western Pipelines, including El Paso Natural Gas Company (“EPNG”).  I am responsible for the pipeline design, mathematical modeling, and transportation capacity planning determinations for EPNG and the other pipelines that form the El Paso Western Pipelines.

Q.	Please briefly summarize your educational background and professional experience.

A.	I earned a Bachelor of Science in Mechanical Engineering from the University of Arizona, 1987.  I am a registered Professional Engineer in the state of Texas.  I began my career with EPNG in 1987, as a project engineer and progressed through various positions in the Engineering, Operations and Field Services groups.  In 1996, I joined Tenneco’s Gas Control group where I worked on various projects to improve pipeline operations.  In 1998, I joined Tennessee Gas Pipeline’s (“TGP”) Facility and Capacity Planning group, where my work included compressor horsepower optimization and various other pipeline design and capacity planning projects.  With the merger of El Paso Corporation and Coastal Corporation in 2001, I became Manager of the American Natural Resources Pipeline Facility and Capacity Planning group, with responsibility for all aspects of facility design and capacity planning.  In 2004, I joined EPNG as Director of Facility Planning.

Q.	Have you previously testified or presented testimony before the Federal Energy Regulatory Commission?

A.	No.

Purpose of Testimony

Q.	What is the purpose of your prepared direct testimony in this proceeding?

A.	The purpose of my testimony is first to explain what Delivery codes (D-Codes) are, how EPNG defines D-Codes, what D-Codes cover, how they are used, and what their shortcomings are.   I will conclude by proposing changes to certain D-Codes that, in EPNG’s operational judgment, are necessary in order to establish the proper balance between shipper flexibility and delivery point specificity.  These changes, which will subdivide certain D-Codes into smaller D-Codes, will allow EPNG to create a workable model for designing capacity additions, contracting to help provide reliable firm transportation to its shippers, and scheduling shipper transportation activity on a daily basis.  The lack of specificity with the current D-Code definitions threatens service reliability, and El Paso therefore believes it is prudent to address this situation now, before shippers experience a loss of firm service.  In addition to proposing D-Code changes, I also will explain and support EPNG’s related proposal to establish a Maximum Delivery Obligation (“MDO”) at each individual meter within a D-Code, which EPNG also believes is operationally necessary in order to ensure the reliability of firm service.

Exhibits

Q.	What exhibits are you sponsoring?

A.	I am sponsoring seven exhibits:

EPG-197  Schematic of the Phoenix area with four overlapping D-Codes

EPG-198  Example of D-Codes spanning multiple laterals

EPG-199  Schematic showing lateral capacity variations

EPG-200  Proposed Phoenix area D-Code diagram

EPG-201  Example lateral with multiple shippers’ distributed loads

EPG-202  Example lateral with multiple shippers’ loads at furthest point

EPG-203  Proposed D-Codes: Improved Definition

Q.	Were these exhibits prepared by you or under your supervision or direction?

A.	Yes.

Analysis

Q.	What is a D-Code?

A.	A D-Code is a delivery meter or a group of delivery meters whose operational impacts on the system should be similar.  The volumes for each meter in a D-Code are aggregated into a single theoretical point for purposes of contracting, nominating and scheduling transportation services.  For convenience and consistency, D-Code names are assigned even to single points of delivery so that shippers can schedule on a consistent basis, using the D-Code rather than meter number nomenclature.  Of the approximately 400 D-Codes in use on the system, about 70 percent represent only a single delivery location.  The other D-Codes include multiple delivery locations within each D-Code.  Measuring size by the number of meters included, the nine largest D-Codes on the system encompass approximately 27 percent of the meters covered by D-Codes, or an average of approximately 27 meters per each D-Code.

Q.	What purpose does a D-Code, this aggregation of multiple meters, serve?

A.	A D-Code simplifies nominations and scheduling for shippers.  As an example, see Exhibit No. EPG-197, which shows one Southwest Gas Corporation (“SWG”) D-Code encompassing over 30 metropolitan Phoenix delivery meters.  Instead of having to figure out the precise meters where its gas needs to flow and then entering multiple values (one for each meter) into the scheduling system, a shipper needs only to nominate a single volume to the D-Code.  The use of D-Codes also facilitates contracting in that shippers can hold their firm transportation rights at the D-Code level rather than at individual delivery meters, simplifying a shipper’s contract with EPNG.

Q.	Why do you say that the lack of specificity in the current D-Codes “threatens service reliability”?

A.	Because, as I will explain, this could result in a situation where shippers take gas in excess of EPNG’s ability to deliver gas to them within the D-Code while honoring its obligations to other shippers, which would require EPNG to curtail service.  When gas is contracted and then nominated at a D-Code which aggregates many delivery meters, it is impossible for EPNG to know where the gas will be taken among the meters in the D-Code.  Shippers’ use of the D-Code effectively designates gas to a general area.  For purposes of determining mainline and lateral capacity, both on a day-to-day operational/scheduling basis and in firm contracting, EPNG must assume some distribution of the D-Code volume among the meters in the D-Code to compute the available mainline and lateral capacity.  Absent a clear definition of delivery rights and obligations, EPNG is left to make an estimate of the distribution which may or may not be representative of the shipper’s needs.  Historically, the state-of-the-art regarding pipeline capacity simulation on EPNG did not support doing transient analysis and tight system design on complicated lateral facilities.  Now that we are approaching the capability to do that sort of work, we can more efficiently and effectively design the pipeline system to insure reliable firm service if we have more precisely defined the shipper’s rights and EPNG’s obligations to serve.  In addition, by achieving a better definition of service locations and service structures, we expect to be able to provide our shippers with new firm service options that include delivery pressure guarantees that we cannot provide in today’s environment.

Q.	Why do these changes need to be addressed right now?

A.	El Paso’s lateral systems were designed and constructed many years ago.  In general, they were built with larger diameter pipe that was more than sufficient to meet shippers’ needs.  As a result, most of the customers have enjoyed a long period during which the lateral capacity has not been an issue even though D-Codes spanned multiple lines in large geographic areas.  However, as loads in the East-of-California (“EOC”) area have grown, usage on some of the laterals has begun to approach their capacity limits.  In other cases, while usage has not yet reached capacity, there is now sufficient load connected that we believe that at certain times usage could spike upwards very quickly. Given the customers’ desires for certainty of flow and the Commission’s recent emphasis on restoring reliable service on EPNG’s system, EPNG believes it must act proactively to address a number of existing D-Codes that have become or are soon projected to become problematic.

Q.	How does the geographical area covered by a D-Code impact the need to moderate existing D-Codes?

A.	The geographical area encompassed by a single D-Code may comprise groups of meters that span multiple laterals.  The largest D-Code on the system currently encompasses about 89 delivery meters as shown in Exhibit No. EPG-198 where shaded regions on the map are used to identify the general area covered by each D-Code.  As the map shows, several D-Codes in southern Arizona span multiple laterals.

Q.	Does EPNG have sufficient capacity to transport its firm contract volumes on the laterals assigned to D-Codes?  How do you reconcile that statement with your description of this problem?

A.	We do have the capacity to deliver all of the gas currently allocated to each D-Code, so long as the following assumptions are used:  (1) the distribution of the shipper’s takes by meter within the D-Code is as we have modeled it based on past experience, (2) the shipper takes gas ratably over the course of the day, and (3) the shipper balances its daily receipt and delivery quantities.  However, we are proactively responding to a forward-looking system need.

Q.	Please continue with your description of the need for D-Code change.

A.	As loads grow, the historical distribution of deliveries may not continue to be a good indicator for future flows.  In addition, in cases where a D-Code involves delivery points on multiple laterals, even though we have enough capacity to handle all of the contractual rights assigned to that D-Code if the volumes are spread over all of the laterals, the aggregate shipper rights may exceed the capacity of any individual lateral line.  Since the current contract structure may give shippers the right to deliver to any point within the D-Code, we cannot guarantee that one shipper’s use of its rights will be compatible with another shipper’s use of its rights.  That could result in a shipper not getting the gas it had scheduled.

Even if the D-Code includes only one lateral, this problem may occur if flow patterns change.  Lateral capacity can vary by 20% or more depending on where along the lateral gas is delivered.  Splitting a delivery volume such that half is delivered to a meter located near the midpoint of the lateral and the other half is delivered to the end of a lateral results in a lateral capacity that is more than 20% greater than delivering all of the gas to the end of the lateral.  Exhibit No. EPG-199 (Schematic of Lateral Capacity) illustrates this issue.  Thus, if shippers were to decide to make all their deliveries to the end of the lateral in this example, EPNG would have 20 percent less capacity on the lateral, which provides one example of why EPNG needs to define more precisely shippers’ rights within D-Codes.  Further exacerbating the issue of lack of definition of delivery location is the fact that multiple shippers may have D-Codes in the same geographic area.  There is delivery obligation overlap, as shown in Exhibit No. EPG-197.

Q.	Is there a problem with shippers having D-Codes that overlap each other?

A.	Yes, there is a problem with overlapping D-Codes.  If we look at the Phoenix area for example, there are four D-Codes that overlap.  Referring to Exhibit No. EPG-197, we see that Arizona Public Service, City of Mesa, Salt River Project and Southwest Gas have overlapping D-Codes.  If one of these shippers decides to take more of its gas at a point where EPNG has not anticipated delivering significant volumes, that lateral(s) could become overloaded.  If that happens, the pressure would fall and the other shippers on that lateral may not receive their firm gas which they rely upon to serve their customers.  With the recent addition of significant gas fired electric generation in this area, the likelihood of such an event occurring is now much higher than in the past.  Additionally, we expect that trend to continue.  For example, the new Santan Power Plant, scheduled to go online in 2005, is capable of taking deliveries of an incremental 140,000 Dth/day.

Q.	Please further explain the problem that can be caused when a shipper decides to take more gas at a location within a D-Code “where EPNG has not anticipated delivering significant volumes”.

A.	Because shippers’ rights within a D-Code are not precisely defined, for planning purposes EPNG instead must rely on historical load patterns, which could change in the future.  Currently EPNG’s Facility Planners design changes to existing infrastructure and estimate the available pipeline capacity by allocating volume quantities to each meter in the D-Code as a component of the overall analysis.  The allocated meter volume is computed through the use of a percentage of the historical average annual meter load determined from the FERC 567 report by dividing individual meter flows by the total volume delivered to meters within that D-Code.  This percentage is then multiplied by the current contractual D-Code volume to arrive at a distribution of the customer’s contract rights that can be used to determine the adequacy of the existing facilities.

Q.	Do you believe history is a reliable allocation method for analysis of future capacity needs under the operating conditions on the pipeline at this time?

A.	Not any longer.  We are trying to be proactive in our analysis of system requirements.  As I noted earlier, in some areas of EPNG’s system, loads have grown to the point that some laterals are running near capacity, some laterals have sufficient load attached that they could easily spike up towards full capacity, while still other laterals remain lightly loaded.  Given the fact that some laterals have a more urgent need to be addressed, we are trying to propose a structure that can be implemented on a priority basis and that is workable for the future.  Delivery volumes to some existing EOC areas are growing at several percentage points per year, new communities are being developed outside of EPNG’s traditional delivery locations and a number of new power plants are coming online.  All of the previously mentioned factors increase the load on portions of EPNG’s system in ways that no one has observed before.  We do not believe historical loads are necessarily indicative of future loads and thus should not be the principal design basis for a firm service.  As discussed in the testimony of EPNG Witness Ms. Catherine Palazzari, we are continuing the transition to more defined rights on EPNG’s system.  In order to ensure we do not again reach the point of routinely administering capacity allocations, we need to break up some of the existing D-Codes.  This break-up will better define the shipper’s needs and EPNG’s obligations so that we can identify potential capacity constraints and respond to each shipper’s current and future needs.  EPNG will not wait until there is a curtailment of firm service to identify a system or service problem.

Q.	If history does not work for predicting delivery point volumes, what does EPNG propose?

A.	EPNG’s planners try to estimate what future needs might be but we have no crystal ball to tell us what is going to happen.  Instead of relying on historical data that we know may not reflect current and future market needs, EPNG proposes to appropriately allow each shipper to establish a more precise definition of its own delivery requirements by making two changes:  first, by subdividing the current D-Codes into smaller groups that better reflect customer needs and EPNG’s system limitations, and second, by defining EPNG’s maximum firm obligations to the individual meter level.  By making these changes, system reliability will be improved.  With these better definitions as to the location, magnitude and quality of its delivery obligations, EPNG also will be able to offer shippers TSAs that include automatic delivery pressure guarantees, which are not available today.

Q.	How do you propose to achieve an improved definition of system rights and capabilities?

A.	From a Facility Planning point of view, to guarantee the firm contracted volume to the desired location, and at the end-user pressure requirements, we would ideally like precise definition of:

- Where the gas enters EPNG’s system (receipt meter)

- Receipt pressure

- Receipt flow rate

- Delivery location - individual meter

- Delivery flow rate (i.e., even hourly, or if uneven then the duration of the peak hour requirements)

- Delivery pressure  (i.e., 100 psig, 250 psig, 500 psig)

From a practical point of view we recognize that shippers may not currently be able to provide all the necessary information with perfect precision due to the difficulty, particularly for an LDC serving the residential/commercial market, of accurately forecasting their loads.  Power plant operators face a similar problem but the impact of forecasting difficulty for electric generators can be tempered by wheeling power such that the whole burden of serving the market does not fall on one operator.  However, all parties must work to prevent forecasting difficulties from creating operational problems for other shippers.  For example, as the California LDCs have shown, with construction of adequate market area storage and LDC delivery facilities, the daily and hourly load variations at the pipeline delivery location can be handled without materially and routinely swinging on the supplying pipeline’s line pack.  In EPNG’s case, because EPNG is so closely integrated with its shippers’ needs through multiple interconnections, EPNG and its shippers have to work out a process for defining the daily and hourly flow requirements by specified location on the delivery laterals.

There are two fundamental ways to address the problem of measuring the delivery lateral capacity at a given level of customers’ demand.  The first and most extreme approach is to assume that all shippers have a right to flow as far down the lateral as possible and to limit the contractual volumes to the resulting minimum.  The second is to precisely identify each shipper’s rights at each individual meter location and require the shipper to nominate gas to each individual meter.  This would add a significant burden to all parties in terms of time and complexity, (for example, the on-going costs of hiring additional schedulers by the shippers), but would have the benefit of maximizing the amount of firm capacity rights that each shipper could hold.

The compromise EPNG proposes, which retains considerable flexibility and yet still provides the surety that firm service requires, involves breaking up the existing D-Codes into smaller geographically and hydraulically related pipe segments to improve deliverability and reliability.  We also propose to define an upper limit on the pipeline’s delivery obligation at each meter within the D-Code, which we propose to call a Maximum Delivery Obligation or MDO.

Q.	Specifically, for what areas is EPNG proposing a D-Code change?

A.	EPNG believes that, if its proposal is not adopted, the following laterals may experience reliability issues related to load growth, overlap of shipper D-Codes and/or the fact that the D-Codes span large geographic areas and multiple laterals:

- Phoenix Area Laterals – Lines 2039, 2072, 2222, 2214

- Anthony Lateral – Lines 2054, 2053

- West El Paso Lateral – Lines 2034, 2058, 2043

- Globe/Miami Lateral – Lines 2004, 2005

- Nogales Lateral – Line 2020

- Twin Buttes Lateral – Line 2143

- Douglas Lateral – Lines 1004, 1005

- Silver City Lateral – Lines 2084, 2008-10, 2086-7, 2089

- Superior Lateral – Line 2026

- Hayden/San Manuel Lateral – Line 2140, 2144

- Ajo Lateral – Line 2025

Q.	Do you believe there is a workable option other than breaking up the D-Codes that encompass points on these laterals?

A.	No.  As to the first option considered, Facility Planning could assume the most hydraulically demanding delivery load distribution (flows pushed to the most distant point on the lateral), but we believe this option would unnecessarily limit firm contract rights.  Moreover, on a day-to-day basis EPNG would not schedule gas, particularly firm secondary transactions or interruptible transportation, that could have otherwise flowed, due to the restrictive definition of firm capacity under this option.  Another option considered could be to leave the system as is and then wait for a curtailment of firm service.  Based on our operational judgment, we do not believe this is a viable option since it would not be a prudent course of action.  Moreover, I believe following that course of delayed action would only be more costly than taking more effective action now, because parties could experience costs associated with the loss of firm service and then would also incur costs to prudently expand the system through piping reinforcements.  It would only then be by analyzing a system failure that you could then identify where the piping reinforcements are needed and what load definitions are necessary.  An additional problem with waiting until you have a loss of firm service is that it could take two years to remedy the capacity shortfall if land or land right acquisitions are required to support expansions.  In addition to these matters, there is the overall policy issue of how one prepares the system to meet tomorrow’s demand if the shippers’ contractual flow rights are not precisely defined.

Q.	Why would it take two years to remedy a system capacity shortfall?

A.	For example in Arizona condemnation is often required for new facility construction and the process usually requires over a year to gain access to right-of-way.  When you add to that the construction time required to deal with what would, in many cases, be urban construction, expanding a lateral can easily take two years.

Q.	Can you provide an example of how EPNG proposes to divide the existing D-Codes into smaller, more workable aggregated delivery groupings?

A.	Yes.  We propose to break the D-Codes up into hydraulically and geographically meaningful meter groups such that there is a minimum of overlap and capacity can be reasonably defined.  The objective is to ensure reliability of service by breaking up D-Codes in a manner that enables EPNG’s planners to define each shipper’s rights in relation to the other shippers in the area while causing the least shipper disruption.  For an example of how this would be done, Exhibit No. EPG-200 shows how we would break up the D-Codes applicable to the Phoenix area laterals.  We believe that the better definition of delivery obligations provides a workable framework for scheduling daily capacity with defined location rights that gives EPNG enough knowledge about system operations that it can meet the firm needs of its customers while still scheduling firm secondary and interruptible transportation.

Q.	Earlier you stated that you wanted to define firm rights to the individual meter level.  Please explain your proposal.

A.	We propose to identify a Maximum Delivery Obligation (“MDO”) at each individual meter within a D-Code.

Q.	Why do you believe it is operationally necessary to have a meter delivery obligation?

A.	The situation within a D-Code is similar to the basic problem with D-Codes.  While it is a customer service feature to group the delivery meters for scheduling purposes, the various meters may not have similar hydraulic consequences.  The further you transport gas in a given pipeline the lower the pressure at the delivery point because of the frictional losses. Pressure drops as a function of velocity squared.  For example if you increase the flow by 10%, holding the upstream pressure into the lateral the same, the downstream delivery pressure falls 21%.  Additionally, meter capacity is dependent on pressure.  As the pressure in the pipeline falls, so does the delivery capacity of the meter.  Essentially, you get hit twice.   If a shipper decides to shift delivery volume to a delivery meter further out on the lateral not only does the lateral capacity decrease but so does the meter delivery capability of all the meters along the flow path.  One shipper’s takes from any one meter can negatively impact other shippers on the same line.

Q.	How would an MDO work?

A.	A MDO defines the maximum amount of a gas that the Shipper and Transporter agree will be delivered at a firm hourly rate to a specific meter based on a minimum pressure defined in the Shipper’s TSA.

Q.	Why is an MDO required?

A.	MDOs enable the transporter and the shipper to accurately determine the lateral capacity, and thereby help to prevent loss of firm service.

Q.	Will you allow an hourly defined MDO right that differs from that defined in the shipper’s TSA?

A.	No.  The TSA defines the contract rights, while the MDO defines maximum deliveries within those rights.  Thus, the MDO rights are consistent with the shipper’s rights under the TSA.

Q.	How do MDOs help with lateral capacity determination?

A.	Knowing how much gas has to travel along a section of pipe to a particular location at a given delivery pressure enables the modelers to construct a mathematical model that can more accurately compute the lateral capacity.

Q.	Do MDOs help the shippers and Delivery Point Operators in any other way?

A.	Yes, an MDO along with a pressure guarantee in the TSA enables the Delivery Point Operator to design its downstream facilities with confidence that service will be sustainable.  If, by better defining the needs and rights of the shipper, the transporter can offer the proper menu of services along with a minimum pressure guarantee, then the shipper (point operator) may not have to spend dollars to improve its facilities because they can take advantage of the guaranteed pressure and firm hourly rights available from the pipeline.  In addition, MDOs can help the delivery point operator determine how much it may need to spend for facility enhancements on its side of the meter to receive the desired level of flexibility and service.

Q.	Can you explain how the contract volumes associated with a D-Code will relate to the MDO limits you have just described?

A.	Yes.  The D-Code will still be used for contracting and scheduling gas.  The maximum quantity of gas that the shipper has a firm right to take at the collection of meters within the D-Code will be defined at that level.  Internal to a D-Code, the MDO limits will specify how much of the total D-Code volume can be taken at a particular meter.  Because a delivery point operator may have multiple meters, the sum of the MDO volumes can be larger than the contracted D-Code volume.  If it is, EPNG will model the lateral’s capacity by assuming flow to the most hydraulically demanding points first.

Perhaps an example will help tie all of this together.  See Exhibit No. EPG-201 and Exhibit No. EPG-202.  Take, for example, a lateral with four delivery points.  Assume there are two shippers, A and B, who want to take gas to that lateral and that the first shipper, A, wants to use the first and third delivery points while the second shipper wants to use the second and fourth delivery points.  Assuming the flow is divided evenly among the four meters, the lateral capacity is 5,000 Dth per hour and we can deliver 1,250 Dth per hour to each meter, as shown on Exhibit No. EPG-201.  If EPNG has to assume that each shipper can take the full volume of their contract to the farthest delivery point at which they have firm rights, we would have to model, sell, and schedule capacity on the basis that all of shipper A’s volume is to be transported to the third delivery point and all of shipper B’s volume has to be transported to the fourth delivery point.  The result in the latter case is that the lateral capacity has been reduced from 5,000 Dth per hour to 3,500 Dth per hour.  Additionally, if a shipper were to assume the right to shift volumes without EPNG’s knowledge, i.e., if Shipper A shifted all of his volume to the third meter, he could pull the pressure so low that Shipper B might not be able to get the gas he was expecting at his meter #4.  If, in contrast, the shippers can tell us that they really only need 25% of their D-Code volume at the most downstream points within their D-Codes, the fact that we deliver more gas off the lateral sooner means that we can move more gas on the lateral in total.

Q.	Do you have a schedule that details how you propose to divide the existing D-Codes into a more manageable system?

A.	Yes.  Please see Exhibit No. EPG-203, Proposed Improved D-Codes.

Q.	Has the proposal regarding D-Codes and MDO’s been reflected in tariff language?

A.	Yes.  The tariff is sponsored by EPNG Witness Mr. Richard Derryberry.

Q.	Can you summarize the advantages of a refined set of D-Codes and MDOs?

A.	EPNG’s believes further refinement of the existing D-Codes and the establishment of MDOs at specific meters will enable to EPNG to provide shippers on its system with firm reliable service that retains flexibility for the marketplace.  The benefits include:

- The continuation of the D-Code aggregation process which significantly facilitates nominations and scheduling as compared to working at the individual meter level,

- Shippers, Point Operators, and EPNG will have clearly defined rights and obligations,

- System reliability will be improved and EPNG can more accurately determine what space is available for all shippers, and

- Better definition of lateral flow requirements will accommodate efficient contracting for specific market loads and avoid construction of excess facilities by either the pipeline or the shipper.

Q.	Does that conclude your testimony?

A.	Yes, this concludes my testimony.