DOCKET NO. RP08-___-000

EL PASO NATURAL GAS COMPANY

NOTICE OF RATE CHANGE

STATEMENT O

EL PASO NATURAL GAS COMPANY

DESCRIPTION OF COMPANY OPERATIONS

_____________________

El Paso Natural Gas Company ("EPNG") is a natural gas company engaged in open-access transportation.  EPNG owns and operates an interstate pipeline system which extends from various natural gas production areas in the southwestern United States through the States of Texas, New Mexico, Colorado, Arizona, and California.  EPNG's northern system extends from production areas in New Mexico and Colorado to a location near Topock, Mojave County, Arizona at the boundary between the States of California and Arizona.   EPNG's southern system extends from production areas in west Texas and southeastern New Mexico to a point of interconnection with EPNG’s Line No. 1903 system near Ehrenberg, Arizona.  In addition to Line No. 1903, EPNG’s northern and southern systems are also interconnected through its Havasu and San Juan Crossover laterals.  EPNG's interstate system makes deliveries to numerous other on-system delivery points and to off-system eastern markets.

As of March 31, 2008, the termination date of the base period used herein, EPNG continues to provide non-discriminatory open-access transportation pursuant to Subparts B and G of Part 284 of the Commission's Regulations under the Natural Gas Policy Act of 1978.

As of March 31, 2008, EPNG’s interstate system was comprised of approximately 10,245 miles of main and branch transmission pipelines and an aggregate of 912,751 (ISO) horsepower of compression.

EL PASO NATURAL GAS COMPANY

DETAILED SYSTEM MAP

_______________________

A detailed system map showing El Paso Natural Gas Company's pipeline system is provided herein.

EL PASO NATURAL GAS COMPANY

MAJOR EXPANSION AND ABANDONMENT CERTIFICATES

ISSUED SINDE EL PASO NATURAL GAS COMPANY’S

LAST GENERAL RATE CASE

_______________________

Docket No. CP06-57-000

On February 1, 2006, EPNG filed an application under Section 7(c) of the Natural Gas Act (“NGA”) for authorization to acquire, own, and operate the East Valley Lateral (“EVL”) facilities comprised of approximately 37 miles of 24" O.D. pipeline, with appurtenances, from the Salt River Project Agricultural Improvement and Power District ("SRP").  The EVL facilities have a free flow capacity of 342,500 Dekatherms per day ("Dth/d") and extend north from an interconnect with EPNG's Line No. 2000 in Pinal County, Arizona to SRP's Santan Power Plant facilities in Maricopa County, Arizona.  As part of the purchase agreement, EPNG agreed to a purchase price of $32 million and granted SRP two "re-purchase options" for undivided ownership interest in the lateral.  The first option granted SRP the ability to re-purchase from EPNG an undivided interest in the EVL equal to a flowing capacity of 203,500 Dth/d for $14 million.  The second option granted SRP an opportunity to re-purchase an additional undivided ownership interest in the EVL equal to a flowing capacity of 39,000 Dth/d from EPNG for $1.  Ultimately, the SRP negotiated an agreement with Transwestern Pipeline Company, L.L.C. ("TW") to sell both re-purchase options to TW, so that TW could incorporate the capacity and related undivided interest in the EVL into its proposed Phoenix Expansion Project.  On April 20, 2006, the Commission granted EPNG authorization to acquire the EVL facilities.  EPNG acquired the EVL facilities from SRP on May 25, 2006.

Docket No. CP06-373-000

On June 2, 2006, EPNG filed an application under Section 7(b) of the Natural Gas Act (NGA) for authorization to abandon, by sale, to West Texas Gas, Inc. (“WTG”) its Snyder and Sonora Supply Lateral Pipeline Systems (Snyder and Sonora Pipelines), with appurtenances, located in Scurry, Borden, Howard, Martin, Andrews, Ector, Sutton, Schleicher, Crockett, Irion, Reagan and Upton Counties, Texas.  The Snyder and Sonora Pipelines comprise a total of approximately 236 miles of various-sized pipeline supply laterals and appurtenant facilities located in the Permian Basin area in various counties in West Texas.  EPNG agreed to sell the two pipeline systems to WTG for approximately $300,000.  By order dated August 1, 2006, the Commission granted EPNG authorization to abandon, by sale, the Snyder and Sonora Pipelines. EPNG conveyed the facilities to WTG on October 1, 2006.

Docket No. CP07-9-000

On October 16, 2006, EPNG filed an application pursuant Section 7(b) of the NGA for permission and approval to abandon, by sale, to TW up to 70 percent of an undivided ownership interest in the EVL, i.e., up to 242,500 Dth/d of the EVL’s current capacity of 342,000 Dth/d.  As part of the EVL purchase and sale agreement, SRP transferred its re-purchase options for an ownership interest in the EVL to TW.  TW notified EPNG of its intent to exercise its option to re-purchase an undivided ownership interest equivalent to a flowing capacity of 203,500 Dth/d for a price of $14 million.  TW has not indicated that it will exercise the second purchase option.  TW intends to incorporate the EVL capacity into the design of its Phoenix Lateral Project which is intended to be in-service by October 2008.  On November 15, 2007, the Commission issued an order approving, among other things, EPNG’s proposed abandonment of an undivided ownership interest in the EVL.  Effective June 30, 2008, EPNG has not transferred the undivided ownership interest in the EVL to TW.

Docket No. CP07-448-000

On September 18, 2007, EPNG filed an application pursuant to Section 7(c) of the NGA and Part 157 of the Commission’s regulations for authorization to construct and operate the Picacho Compressor Station on its existing EVL in Pinal County, Arizona.  The Picacho Compressor Station is designed to increase the EVL’s capacity from 342,500 Dth/day to 372,500 Dth/day.  Subject to the exercise of the TW re-purchase options mentioned above, EPNG will be able to provide up to 130,000 Dth/day of service on the EVL once the Picacho Compressor Station is placed into service.  Thus, construction of the compressor station will ensure that EPNG is able to maintain existing services and also provide EPNG with additional flexibility for service in the Phoenix area, particularly during peak demand periods.  EPNG estimates that the Picacho Compressor Station will cost approximately $23,899,700.  On February 15, 2008, the Commission issued an order authorizing the construction and operation of the Picacho Compressor Station.  Construction of the compressor station is currently underway.

Docket No. CP08-2-000

On October 2, 2007, EPNG filed an application pursuant to Sections 7(b) and 7(c) of the NGA, requesting authorization to relocate and replace its Eunice Compressor Station located in Lea County, New Mexico and for permission and approval to abandon the existing Eunice Station.  In order to comply with stricter State of New Mexico air emission requirements, EPNG was required to replace existing reciprocating compressor engines at its Eunice Compressor Station.  Based on various factors, EPNG determined that the best alternative would be to relocate and replace the Eunice Station.  In order to match current system requirements, EPNG opted to install two Taurus 60 compressor units at the new station with a combined site rating of approximately 11,738 horsepower. The new station would be located directly adjacent to its existing Eunice “B” Station, which is approximately 1,700 feet upstream of the existing Eunice Station.  The estimated cost of the proposed facilities is approximately $24 million.  On April 15, 2008, the Commission authorized the construction of the new Eunice Station.  Construction of the compressor station is currently underway.

Docket No. CP08-14-000

On November 1, 2007, EPNG filed an application under Section 7(c) of the NGA for a certificate of public convenience and necessity seeking authorization to construct and operate the Hobbs Expansion Project facilities near the Town of Hobbs in Lea County, New Mexico.  The facilties which are comprised of a new 3,550-horsepower Caterpillar G3612 gas-driven reciprocating jumper compressor unit, a 7.3 miles 20-inch diameter lateral, and a meter station were designed to provide service to an electrical generating facility owned and operated by Southwestern Public Service Company.  Additionally, the proposed facilities also were designed to provide general system benefits that result in compressor fuel savings.  On April 30, 2008, the Commission issued an order authorizing the construction and operation of the Hobbs Expansion Project facilities.  Construction of the compressor station is currently underway.

EL PASO NATURAL GAS COMPANY

Description of System Design and Operation

_______________________

SYSTEM DESIGN

EPNG has traditionally designed its pipeline system to meet its firm contractual demands using the Fundamental Flow Equation in the SynerGEE Gas V.3.6 steady state pipeline simulation tool from Stoner Associates, Inc.  The market’s need for non-ratable takes required a transition to services that explicitly provide for uneven hourly flows.  As a result, EPNG is also using the Unsteady State Module, also provided by Stoner, for analysis of its system.  The unsteady state module utilizes the same Fundamental Flow Equation (described below) but adds the differentiation to reflect changing values over time.  Volumes shown on the flow diagrams are expressed in millions of cubic feet measured at 14.73 p.s.i.a. and at 60° F.

The Fundamental Flow Equation, which describes the flow of natural gas in pipelines, including the effect of elevation changes, is as follows:

Q = 77.54 TbPbD2.5P12P220.0375G (h2−h1)Pa2ZTaGTaLZf0.5	Q	= Gas flow in CF/d

Pb	= Pressure base, p.s.i.a.

Tb	= Base temperature, °R

D	= Internal pipe diameter, inches

e	= Efficiency factor (Dimensionless = 1)

P1	= Inlet pressure, p.s.i.a.

P2	= Outlet pressure, p.s.i.a.

G	= Specific gravity

Z	= Compressibility factor, Standing-Katz-modified Benedict-Webb-Rubin equation

h2-h1 	= Change in elevation, feet

L	= Length of pipeline, miles

Pa	= Average pressure, p.s.i.a. (2/3[P1+P2-P1P2/(P1+P2)]

Ta	= Average flowing temperature, °R

f	= Friction factor

1fkd3.7+2.51Ref	The friction factor is calculated using the Colebrook-White Friction Factor Equation:

where

k	= Roughness, inches

d	= Internal pipe diameter, inches

Re	= Reynolds number

Stoner’s “Actual Horsepower Equation in the Theoretical Form (HP)” is used to model the compression. Compressor performance estimated by the network simulator is verified using performance curves supplied by the compressor manufacturers. The equation is;

HP = QZ K1PdPsK3 - K2TTbwhere

HP	= Horsepower

Q	= Flow (MMscf/d)

Tb	= Base temperature (°R)

T	= Flowing temperature (°R)

Z	= Suction compressibility (dimensionless), Standing-Katz-modified Benedict-Webb-Rubin equation

Pd	= Discharge pressure (p.s.i.a.)

PS	= Suction pressure (p.s.i.a.)

K1	= Arbitrary constant (HP/MMcfd)

K2	= Arbitrary constant (HP/MMcfd)

K3	= Arbitrary constant

EPNG derates its compressors to reflect actual elevation and ambient temperature at the location. Constants K1 and K2 are adjusted so that the theoretical equation approximates manufacturer's performance data.

Ambient temperature assumptions are based on historical meteorological data for the location and season being simulated.  The table below reflects the ambient temperature assumptions currently in use for each of EPNG’s mainline compressor stations during winter and summer months:

| Station | Summer Ambient º F | Winter Ambient º F |
| --- | --- | --- |
| Afton | 79.0 | 44.4 |
| Alamo Lake | 90.0 | 52.3 |
| Amarillo | 76.3 | 37.8 |
| Belen | 76.1 | 36.8 |
| Black River | 80.4 | 46.0 |
| Blanco | 73.0 | 31.4 |
| Bluewater | 68.4 | 32.7 |
| Bondad | 65.5 | 26.0 |
| Bowie | 81.2 | 47.2 |
| Caprock | 76.0 | 40.9 |
| Casa Grande | 87.7 | 51.9 |
| Castle Dome | 86.7 | 50.3 |
| Cimarron | 81.2 | 47.2 |
| Cornudas | 77.5 | 43.6 |
| Deming | 78.0 | 43.3 |
| Dilkon | 72.7 | 33.5 |
| Dimmitt | 74.2 | 36.4 |
| Dumas | 76.2 | 36.1 |
| Dutch Flat | 83.7 | 49.9 |
| EPNG | 82.2 | 47.0 |
| Eunice | 79.0 | 44.4 |
| Flagstaff | 63.1 | 30.3 |
| Florida | 77.9 | 42.3 |
| Gallup | 72.1 | 33.4 |
| Gresham | 77.4 | 44.4 |
| Guadalupe | 77.7 | 46.6 |
| Hackberry | 80.1 | 44.4 |
| Hueco | 79.9 | 45.3 |
| Keystone | 81.9 | 45.6 |
| Laguna | 72.5 | 35.2 |
| Leupp | 73.8 | 33.6 |
| Lincoln | 66.5 | 32.2 |
| Lordsburg | 78.8 | 43.4 |
| Monument | 81.9 | 45.6 |
| Navajo | 71.5 | 33.3 |
| Oracle | 78.1 | 46.7 |
| Pecos River | 80.4 | 45.1 |
| Plains | 76.9 | 41.4 |
| Puckett | 81.5 | 48.3 |
| Rio Vista | 73.0 | 31.4 |
| Roswell | 79.4 | 42.4 |
| San Juan Mainline | 72.6 | 32.2 |
| San Simon | 79.4 | 45.1 |
| Schafer | 79.0 | 40.2 |
| Seligman | 70.3 | 37.6 |
| Tom Mix | 78.1 | 46.7 |
| Topock | 86.3 | 49.7 |
| Tucson | 86.4 | 54.3 |
| Vail | 84.9 | 52.8 |
| Waha | 82.2 | 45.7 |
| Washington Ranch | 80.4 | 45.1 |
| Wenden | 86.7 | 50.3 |
| White Rock | 69.8 | 30.0 |
| Willcox | 76.1 | 43.4 |
| Williams | 65.9 | 33.5 |
| Window Rock | 67.0 | 30.4 |
| Wink | 82.0 | 45.6 |

Notes:  Summer ambient temperature is an average of June, July and August.  Winter ambient temperature is an average of December, January and February.

For design work, receipt and delivery locations and quantities are based on shippers’ firm contractual rights.  Receipt pressures are based on historical operations.  Delivery pressures are based on contractual commitments.  Ground temperatures, pipeline roughness, gas composition, heat transfer coefficients, pipeline efficiency, etc. are based on engineering estimates and are correlated to observed pipeline flows to ensure that, in aggregate, they reasonably reflect actual operating conditions.  Modeling is typically performed for both winter and summer operations.  When transient modeling is performed, assumptions are made as to the variation in hourly rate of flow for each receipt and delivery location.  Typical load duration curves for an electric power plant load and a local distribution company load are shown below.  As these curves demonstrate, deliveries to the interconnects at the California border are typically ratable while the EOC load profile varies seasonally.  During the winter months, the load profile reflects a traditional residential/commercial demand pattern with two peak flow periods, one in the morning and one in the evening.  During the summer months, the load profile is more aligned with the electrical generation market and is dominated by a single mid-afternoon load peak.

January LDC Load Profile

July Generator Load Profile

EPNG offers an interruptible “Pooling” service which allows the aggregation of gas from multiple physical interconnects for delivery to other Poolers or to shipper’s transportation service agreements (“TSAs”).  Pooling is so widely used that few firm shippers elect to hold their receipt rights at physical receipt points.  Instead most choose to hold their receipt rights at the “Pooling Area” level.  There are four such pooling areas in the San Juan Basin, three in the Permian Basin and one in the Anadarko Basin supply area.  This system of contracting for capacity significantly facilitates the transportation of gas but does not provide a solid definition of specific locations at which gas will be received into the system.  Therefore, for design purposes, EPNG assumes gas equaling shippers’ aggregate firm receipt rights will be received at locations that optimize the use of the pipeline facilities within each pooling area.

EPNG allows shippers to aggregate multiple delivery locations into what is known as a D-Code and to hold their firm delivery rights at this aggregated level.  These delivery codes facilitate scheduling by allowing a shipper the opportunity to treat multiple physical locations as though they constituted a single delivery point.   For most mainline system design purposes, EPNG models the firm shippers’ delivery rights at physical points within the D-Code based on contract meter quantity (CMQ).  The CMQ is also used in the analysis of laterals.

SYSTEM OPERATION

General Comments - EPNG’s system receives gas in three supply basins, the Anadarko, Permian and San Juan and delivers it to markets in Texas, New Mexico, Arizona, Nevada, Mexico and California.   Gas flow is predominantly east to west and north to south although there are two notable exceptions to this general rule.

These exceptions are related to what is frequently called the “East End” business wherein EPNG moves gas from the San Juan and Permian Basins to delivery points in the Permian and Anadarko Basins.  While some of this business is done under long term firm contracts, much of it is done on an interruptible or short-term firm basis as opportunities present themselves.

- The facilities in the panhandle of Texas are bidirectional and can be used to bring Anadarko gas down to Plains for further transport to western markets or to move San Juan and Permian Basin gas up into the Anadarko for delivery to mid-continent markets.

- Many of the intrastate pipeline receipt facilities in the Waha area of the Permian Basin are bi-directional.  A significant portion of EPNG’s East End business involves making deliveries into these pipelines for transport into the Texas Intrastate market.

North System Operation – Historically San Juan Basin supplies (including volumes received into that Basin from Rocky Mountain supply basins via Northwest Pipeline and TransColorado) tend to be base loaded with the swings in demand taken by Permian Basin supplies.  Within the Permian Basin, gas tends to be sourced first from the Keystone and Plains Pooling Areas and second from the Waha Pooling Area.  These receipt preferences cause EPNG to preferentially load its North Mainlines (the system between the San Juan Basin and the California border at Topock) in an attempt to move as much gas as possible to the Arizona and California markets along this shorter route.  This, in turn, causes the base loading of the Havasu Crossover and Maricopa Lines flowing from north to south.  The Permian-San Juan Crossover takes the swing in the north to south load.  Gas moved down this crossover is pushed through Plains Compressor Station and into the Southern Low Pressure System for transport to East End or west flow markets.

Gas flowing south on the Havasu Crossover is typically turned west at Wenden and sent towards Ehrenberg.  However, when demand in those markets is low, this gas can be turned east and moved into the EOC market area.

Flow on the Maricopa Line is largely dictated by the demand on the north side of Phoenix.   While this line does eventually connect to another lateral that is tied to the Southern Low Pressure System, it is not a true crossover.  The Phoenix market is fed from the north by the Maricopa Line and from the south by Lines 1007, 1008, 1124, and the East Valley Lateral.

Impact of Reliance on Displacement - In addition to the physical north to south capacity described above, some north to south capacity is created by displacement.  Volumes received in the Permian Basin on the south system and destined for delivery to points on the north system are used to satisfy requests for delivery of San Juan gas to south system markets and vice versa.  The level of displacement capacity created by these offsetting transactions varies from day to day and cannot be reliably predicted in advance.  In addition, not all displacement transactions are equally beneficial.  For example, gas scheduled from Permian to points near Topock is always useful in creating an opportunity to move additional San Juan gas to the south system.  However, while gas scheduled from Permian to the Anadarko will always create displacement capacity through the Plains south constraint, it does not create displacement capacity through the south flow constraint at Lincoln station.

Impact of Increasing Rocky Mountain Supplies – The increasing availability of Rocky Mountain supplies has both a direct and indirect impact on the demand for EPNG’s north to south transportation.  The direct effect is that as more gas is made available in the northern Rockies, it tends to find its way into the San Juan Basin through Northwest Pipeline and TransColorado Pipeline.  This increases the supply available to be moved from north to south.  The indirect effects are tied to the construction of the Rockies Express pipeline and its impact on the demand for San Juan and Permian gas in the mid-continent market.  With Rockies Express, there is likely to be less demand for San Juan and Permian gas in the mid-continent market.  A decrease in San Juan gas flowing to this market makes it likely that more of that gas will seek markets along EPNG’s southern system, increasing the demand for north to south transportation.  A decrease in demand for Permian gas in the mid-continent market would reduce the transactions available to create displacement capacity through the Plains south constraint.

Line 1903 is part of the California portion of the facilities used to construct EPNG’s Line 2000 expansion.  The Line 1903 project put into gas service approximately 88 miles of the abandoned 30" O.D. oil line and involved building a 6.4- mile crossover consisting of a 30" O.D. pipeline to connect it to the existing Mojave Pipeline near Cadiz, California.  The line terminates at an interconnection with EPNG’s existing facilities at Ehrenberg.  EPNG acquired transportation capacity on Mojave and uses that capacity to allow its shippers to move more gas from its north system to its south system.

In addition to eliminating most of the reliance on displacement to provide North to South capacity, the Line 1903 Project allowed shippers to access new gas supplies.  Line 1903 interconnects with Mojave, which in turn connects to Kern River Gas Transmission (“Kern”) at Daggett.  Rocky Mountain supplies transported by Kern to Daggett can be scheduled to EPNG markets by way of Mojave and Line 1903.

South System Operation - Within the Permian Basin, gas from the Plains and Keystone pools tends to consistently load the Southern Low Pressure System and Line 2000 facilities.  The High Pressure System from Waha to Cornudas is bi-directional.  Flow direction in this line is determined by strength of demand to off-system Texas intrastate markets relative to demand on the EPNG system.  When EPNG market demand is strong, the line loads up with westbound supplies originating from Waha.

West of Cornudas Station, gas is shifted to preferentially load the High Pressure System which runs from Waha to Ehrenberg with very few loads taken off en-route.  This provides a baseload supply of gas to help serve the California market at Ehrenberg.  The EOC markets in west Texas, New Mexico and Arizona as well as the Mexican markets in Chihuahua and Sonora are typically served by a combination of the Low Pressure and Line 2000 facilities.

When Line 1903 was placed into operation, flows on EPNG’s southern system changed.  At full load, gas continues to flow from east to west and through Wenden station to reach the markets at Ehrenberg.  At lower loads, less gas is need to be pushed west through Wenden and more of the gas moved down the Havasu crossover will be turned east and moved into the Phoenix area markets.

Impact of LNG - Long term, the need to move gas east from Wenden may increase if one or more of the proposed liquefied natural gas (“LNG”) importation projects on Mexico’s west coast are completed and LNG begins to flow north and east on the North Baja Pipeline. That gas may seek markets in southern Arizona, which would increase the east flow from Wenden.  Alternatively, it may seek markets in northern California and cause the reversal of flow direction on Line 1903, with gas moving from south to north.

Washington Ranch - Washington Ranch, EPNG’s only storage field, is operated as an auxiliary line pack facility to help dampen swings created by imbalances and to provide storage and park and loan services.  Located on the east end of EPNG’s Southern Low Pressure System, Washington Ranch is used to help manage the impact of daily imbalances.

- EPNG constructed its Line No. 1903 system in 2005 pursuant to the Commission’s authority granted in Docket No. CP05-2-000.  This line interconnects EPNG’s northern and southern system, and extends from an interconnection with EPNG’s southern system at a location near Ehrenberg, La Paz County, Arizona to an interconnection with Mojave near the Town of Cadiz, in San Bernardino County, California.  EPNG holds a firm capacity contract on the Mojave pipeline system to effectively integrate the Line No. 1903 with its Northern and Southern systems.

SRP is a political subdivision of the State of Arizona, organized and existing under and pursuant to the provisions of Title 48, Chapter 17, Arizona Revised Statutes, having its principal place of business in Maricopa County, Arizona.  SRP operates electric power generating facilities and provides electric service to about 858,000 residential, commercial, industrial and agricultural power users in a 2,900 square mile service territory in parts of Maricopa, Gila, and Pinal Counties, Arizona, as well as to wholesale and mining loads in an adjacent 2,400 square mile area in Gila and Pinal Counties.  SRP burns natural gas in its Kyrene, Agua Fria and Santan electric generating plants in Maricopa County and its Desert Basin electric generating plant in Pinal County.