Northwest Pipeline Corporation

Docket No. RP06-

Exhibit No. NWP-2

Prepared Testimony of

Nancy W. Schultz

Q.	Please state your name, occupation, and business address.

A.	My name is Nancy W. Schultz.  I am Vice President of Technical Services, Williams Gas Pipeline, for The Williams Companies.  My business address is 2800 Post Oak Blvd., Houston, Texas 77056.

Q.	Please provide your educational and business experience.

A.	I received a Bachelor of Science degree in Civil Engineering from The Pennsylvania State University in 1978.  I am licensed to practice as a Professional Engineer in 12 states.   I started my career as an engineer in the petrochemical division of Brown & Root.  In 1982, I began working at Transcontinental Gas Pipe Line Corporation (“Transco”) as Design Engineer.  As Design Engineer, I was responsible for design and coordination of civil, structural design and permitting activities associated with the construction of onshore and offshore pipelines, compressor stations, and metering and regulating stations.  Since beginning at Transco, I have held a variety of positions within Williams Gas Pipeline.  These positions include Manager of Civil Design, Manager of Field Projects, Director of Engineering and Construction Services, Director of Project Management, Vice President – Gulfstream Project, and my current position.  As Vice President of Technical Services, I am responsible for engineering, construction, land acquisition, procurement, technical support (pipeline integrity, compression, controls, telecommunications, measurement and storage), facilities planning and environmental, health and safety for Williams Gas Pipeline’s natural gas pipelines, including Northwest Pipeline Corporation (“Northwest”).

Q.	What is the purpose of your testimony in this proceeding?

A.	I will primarily discuss Northwest’s Capacity Replacement Project and pipeline reliability and integrity.

Q.	Do you sponsor any exhibits?

A.	Yes.  I sponsor Exhibit No. NWP-3, which includes Schedule C-2 – Plant Additions (Major) by Work Order.

Capacity Replacement Project

Q.  	Please describe Northwest’s Capacity Replacement Project.

A.	Northwest’s existing mainline system along the Interstate 5 corridor in the state of Washington includes 268 miles of 26-inch diameter pipeline, and is fully looped with 30-inch diameter pipeline and partially looped with 36-inch diameter pipeline.    The Capacity Replacement Project is the replacement of the 268 miles of 26-inch pipeline with 80 miles of 36-inch pipeline and compression between Sumas and Washougal, Washington.

Q.	Why has Northwest undertaken this project?

A.	In 2003, Northwest’s 26-inch pipeline experienced two breaks in the Sumas to Washougal corridor.  In response to the breaks, Northwest idled the 26-inch pipeline until its integrity could be assured.  Later, the Department of Transportation’s Pipeline and Hazardous Material Safety Administration (“PHMSA”) (formerly the Office of Pipeline Safety) issued a Corrective Action Order in May, 2003, and an Amended Corrective Action Order (“ACAO”) in December, 2003.  The ACAO required that Northwest maintain a reduced pressure on the 26-inch pipeline until subsequent testing justified a return to normal operating pressure, abandon the 26-inch pipeline within 3 to 10 years (3 years for high consequence and Class 3 areas, and up to 10 years for other areas) and install replacement facilities.

To comply with the ACAO requirements, Northwest hydrostatically tested 111 miles of its 26-inch pipeline and received PHMSA approval to temporarily place the tested segments back into service.  These temporarily restored sections of 26-inch pipeline allowed Northwest to meet shipper needs until permanent replacement facilities could be installed.

Q.	What steps did Northwest take to optimize the design of the facilities needed to replace the contracted capacity associated with the 26-inch pipeline?

A.	Through a reverse open season procedure, Northwest offered existing customers the option of returning contracted-for capacity to Northwest if operationally the relinquished capacity could be used to reduce the need for and costs associated with constructing additional facilities and if the relinquishment would not adversely affect the rates of other customers.  Under these procedures, Northwest accepted the relinquishment of 13,000 Dth/day along with the payment of a turnback fee from TransAlta as more fully discussed by Northwest Witness Laren Gertsch. The relinquishment and associated turnback fee reduced the scope and cost of facilities that Northwest had to build in order to effectively replace the remaining capacity associated with the 26-inch line that was required to be abandoned under the ACAO.  In addition, as Northwest began planning the Capacity Replacement Project, it looked to develop a plan that would provide timely replacement of needed capacity, minimize costs, and minimize environmental and landowner impacts.  It was determined that in lieu of replacing all 268 miles of 26-inch pipe with like-kind pipe, Northwest could replace the 26-inch pipeline with fewer miles of higher pressure, 36-inch pipeline loops, and that by adding compression at existing compressor stations, Northwest could restore the necessary capacity to meet its contract demand.

Q.	Did the Commission approve Northwest’s Capacity Replacement Project, as proposed?

A.	Yes.  Northwest submitted updated system flow studies that show Capacity Replacement Project facilities are the optimal combination of compression and looping to provide the capacity to meet Northwest’s firm transportation commitments, allowing appropriate operational flexibility and reliability.    In its Preliminary Determination on Non-Environmental Issues, 111 FERC ¶ 61,293 at p. 62,273 (2005), as well as in its Order Authorizing Abandonment and Issuing Certificate, 112 FERC ¶ 61,269 (2005), the Commission confirmed Northwest’s flow studies and facility design to restore capacity.

Q.	What are the projected costs for the Capacity Replacement Project?

A.	The costs for the Capacity Replacement Project which are projected to be spent by December 31, 2006, after considering cost savings that result from the relinquishment of capacity in connection with the project, are $331 million.

Q.	When do you expect the Capacity Replacement Project to be placed in service?

A.	The Capacity Replacement Project is projected to be placed in service by November 1, 2006.  This schedule meets the requirements that were set forth in the ACAO.

Pipeline Reliability and Integrity

Q.	Please describe the work that Northwest performs to maintain the reliability of its system.

A.	Northwest has an ongoing program to assess the integrity of its pipeline system and address potential problems to avoid disruption of service and potential hazards to people or property.  Among other things, Northwest utilizes cathodic protection to prevent corrosion of its pipelines, internal pipeline inspection tools, hydrostatic testing, and maintenance and operating practices such as pipeline patrols to ensure pipeline integrity and reliability.

Q.	What is needed to prepare pipeline segments for internal inspection?

A.	Depending on the type of inspection to be performed, preparation may include the installation of launchers and receivers for internal inspection tools and the replacement of portions of the pipeline that is incompatible with internal inspections (e.g., short radius degree bends or reduced port valves).

Q.	What type of remediation actions does Northwest pursue?

A.	When needed, remediation methods may include pipeline recoating, the installation of a composite or steel sleeve, or a replacement of a portion of the pipeline.

Q.	How does Northwest minimize disruptions during scheduled and unscheduled maintenance?

A.	Northwest’s pipeline looping and system storage aid in minimizing service disruptions during scheduled and unscheduled maintenance and repairs.

Q.	Has there been any new legislation focusing on pipeline requirements for safety and integrity?

A.	Yes.  Congress promulgated new safety standards in the Pipeline Safety Improvement Act of 2002.  Pursuant to this Act, PHMSA promulgated regulations that require pipeline companies to develop and implement an integrity management program for segments of pipeline in “high consequence areas” as defined in PHMSA regulations.  As noted below, these areas are generally those where pipeline facilities are located in close proximity to densely populated areas, areas of congregation, or other buildings or structures where inhabitants have limited mobility.

Q.	Please describe Northwest’s integrity management program as it has been established to meet PHMSA’s requirements under the Pipeline Safety Improvement Act of 2002.

A.	Northwest’s integrity management program is a systematic approach to monitoring, inspecting, maintaining and repairing the pipeline system developed to ensure the long-term serviceability of the pipeline and to meet the requirements established by PHMSA.

Under Northwest’s integrity management program, Northwest must incur costs to (1) prepare and implement a program, (2) identify all high consequence areas on the pipeline, (3) develop and maintain a recordkeeping system to document program implementation and activities, (4) prepare affected pipeline segments for assessment, (5) assess affected pipeline segments, (6) develop and perform remediation actions to correct any identified conditions which could threaten the pipeline’s integrity, (7) continually assess all high consequence areas, and (8) integrate all pipeline data.

Q.	What timeline must Northwest comply with to satisfy these new federal regulations?

A.	Northwest was required to have a framework for compliance with the new PHMSA regulations in place by December 17, 2004.  Northwest met that requirement.  Northwest published revised procedures related to its integrity management program.  The integrity management program incorporates many of the procedures that Northwest had already been following as part of its ongoing commitment to ensure the safe and reliable operation of its pipeline facilities while also addressing new requirements imposed by PHMSA in response to the Pipeline Safety Improvement Act of 2002.

Q.	How does Northwest identify its high consequence areas?

A.	High consequence areas are based on population density as defined in 49 CFR 192.903.  Structures within Northwest’s right of way or within the potential impact radius are documented on aerial photographs, counted, and verified, if needed, on the ground.

Q.	What are the requirements relating to high consequence areas that Northwest must meet as it implements its integrity management program?

A.	PHMSA regulations require that pipelines determine which threats exist to segments in high consequence areas and that a risk assessment be completed which includes dividing the high consequence areas into the highest-risk fifty percent and the lowest-risk fifty percent.  Pipelines must complete integrity assessments of the highest risk fifty percent by December 17, 2007 and all other high consequence areas by December 17, 2012.  The regulations also require these facilities to be reinspected on seven-year intervals.  The spending levels required to meet these new regulations have been significant.  Such spending levels will continue to be experienced in the future as Northwest works to comply with all of the ongoing requirements of the Pipeline Safety Improvement Act of 2002 and its associated regulations.

Q.	Under the integrity management program, how does Northwest inspect its pipeline?

A.	Federal regulations related to risk assessments in high consequence areas allow for three types of inspection methods:  external direct assessment, in-line inspection and hydrostatic testing.  Northwest makes use of all three methods.

Q.	Have there been any recent accounting pronouncements that have affected the way pipeline inspection and assessment will be recovered in Northwest’s rates?

A.	Yes.  As explained more fully by Northwest Witness Rand Clark, the Commission has recently issued an Order providing guidance as to whether certain pipeline assessment costs, including those done under an integrity management program, will be treated as either capital costs or expenses for ratemaking purposes.  This Order became effective January 1, 2006.  Based on an examination of costs relating to certain inspection and assessment activities which were incurred in the first nine months of the base period as capital spending, Northwest has determined that $5,878,267 should be treated as expense pursuant to the Order.  Another $62,456 was expensed during the first three months of 2006.

Q.	Are these total expenses of $5,940,723, as reflected on Schedule H-1 representative of recurring costs?

A.	Yes they are.

Q.	What capital expenditures is Northwest making, which are necessary to maintain the reliability and integrity of the Northwest system?

A.	The total general system transmission capital expenditures during the test period, excluding the Capacity Replacement Project, total $128,460,916 and are reflected on Schedule C-2.  Schedule C-2 lists all major current capital projects by work order number.    Each of the listed projects is anticipated to be placed in service prior to December 31, 2006, the end of the test period.

Q.	Of these capital expenditures reflected on Schedule C-2, what are the expenditures required to comply with the Pipeline Safety Improvement Act of 2002?

A.	Within the projects described on Schedule C-2, there is $29,352,894 for projects required to comply with this Act.

Q.	Does that conclude your testimony?

A.	Yes, it does.

Northwest Pipeline Corporation

Docket No. RP06-

Exhibit No. NWP-3

Schedule C-2

Plant Additions (Major) by Work Order

[This exhibit is included in Volume I]