Northwest Pipeline Corporation

Docket No. RP06-

Exhibit No. NWP-6

Prepared Testimony of

Barrie B. McCullough

Q.	Please state your name, occupation, and business address.

A.	My name is Barrie B. McCullough.  I work as a natural gas industry consultant.  My address is 2658 E. Willow Wick Dr., Sandy, Utah 84093.

Q.	Please provide your educational and business experience.

A.	I received a Bachelor of Science in Industrial and Management Engineering from Montana State University in 1969.  The following year I received a Bachelor of Science in Aerospace and Mechanical Engineering with a Minor in Economics, also from Montana State University.  In 1970, I began working for Continental Pipeline Company.  While at Continental, I held operations and project engineering positions that included both field assignments and corporate headquarters assignments.  I worked as a project engineer on petroleum products pipeline facility modifications and upgrades, and I worked on new pipeline feasibility study groups.  In 1973, I began working for Montana Power Company.    As a gas engineer at Montana Power, I completed project assignments that included implementation of new industry standards, planning, budgeting, utility economics, project development, facility design and project management.  I completed the design and supported construction of new gas field facilities.  This included all well site facilities, gathering lines, compression, and a transmission line.

I worked for Northwest Pipeline from 1977 to 2001.  As a project engineer at Northwest, I designed grass roots gas gathering compressor stations, and I supervised additions or modifications to existing gas transmission compressor stations.  I held multiple manager and director positions, including Manager of Plants and Stations Engineering, Operation Area Manager, Manager of Transmission Operations, Manager of Projects, and Director of Engineering.  My responsibilities in these positions included oversight of FERC-mandated separation of gas transmission business from gas gathering and processing, supervision of the design and construction of major expansions to transmission facilities, budgeting, and assuring compliance with regulatory standards.  During this time, I also directed and coordinated the preparation of the engineering study used to determine the net negative salvage value for Northwest’s rate case in Docket No. RP96-367.

Since retiring from Northwest in 2001, I have worked as a consultant.  As a consultant, I worked as a site inspector for construction of a compressor station, completed a net negative salvage study for another FERC-regulated interstate gas pipeline company, and managed the design and construction of erosion control features along the pipeline right of way of a major gas transmission line.

Q.	What is the purpose of your testimony?

A.	I will provide testimony about the study I have completed concerning Northwest’s net negative salvage value.

Q.	Please describe the net negative salvage study you conducted.

A.	The study was conducted to determine the costs to abandon or remove all of Northwest’s facilities and the salvage value of the system assets.  The results of the study show that the costs to abandon or remove the system facilities will be greater than the salvage value, creating a net negative salvage.  I provided these results to Northwest Witness Edward Feinstein who uses them to generate a net negative salvage rate for the Northwest system and incorporates them into his discussion of net negative salvage.

Q.	What exhibits are you sponsoring?

A.	I am sponsoring Exhibit NWP-7, Northwest’s Net Negative Salvage Study (“Study”).  The Study consists of eight schedules detailing the assumptions upon which I based the Study, the methods I used to arrive at cost and value amounts, and the final abandonment costs and values I assigned to all of the Northwest system components.

Q.	Please provide a brief overview of the schedules included in the Study.

A.	Schedule 1 lists the assumptions I used to complete the Study.  Schedule 2 shows the costs and credits associated with the abandonment of all of the components of the Northwest system.  Schedules 3 through 8 each provide a summary of the costs and credits associated with the abandonment of the individual components of the Northwest system.

Q.	Please explain the basis of your Net Negative Salvage Study.

A.	I followed requirements from the Department of Transportations rules and regulations for the abandonment of pipeline facilities under Section 192.727 of Title 49 of the Code of Federal Regulations.  I also followed the Gas Piping Technology Committee guide material for recommended procedures for planned shutdowns in connection with abandonment or inactivation of a pipeline.

Q.	What was the scope of your Study?

A.	I looked at the various methods for abandoning the Northwest system necessary to meet the above-referenced legal and regulatory requirements.   I also applied environmental, safety and landowner requirements to determine which facilities can be abandoned in place, which facilities will have to be removed and what remediation efforts will be required.  In order to develop the costs for each element of the scope of work, I used various internal Northwest and external industry technical resources.

Q.	How were the various components of Northwest’s system analyzed?

A.	I divided the system into three categories:  transmission facilities, underground storage facilities (Jackson Prairie), and other storage facilities (Plymouth LNG).

Q.	Let’s begin with transmission facilities.  What approach did you take to determine the costs of abandoning the transmission facilities?

A.	To address the unique requirements associated with different parts of the transmission system, I divided the system into three facility types:  pipeline, compression, and measurement.  Additionally, as discussed later, I have included communications and overhead as part of the transmission facilities.  The costs and credits associated with the abandonment of the transmission facilities are shown in Schedule 3 of the Study.

Q.	Please describe your approach to calculating the costs associated with the abandonment of the pipeline.

A.	To determine the costs of abandoning the pipeline, I began with the general assumption that Northwest will follow the most cost-effective method for handling the abandoned line by abandoning all belowground facilities in place.  To be abandoned in place, the lines must be purged of all gas, cleaned, and then filled with nitrogen.  Some sections of the pipeline, however, have unique requirements that cannot be fulfilled through this abandonment in place process.   Accordingly, Northwest will have to take more extensive measures for these sections of the pipeline.

To determine how different sections will have to be treated, I worked with the Northwest Land Department to determine how many easements specifically require removal of the pipeline upon abandonment.  The Land Department reviewed existing easement and license agreements and determined that, for the most part, the only agreements that specifically require removal of the pipeline upon abandonment are for easements on public lands or under railroads.  Where the agreements for public lands do not specifically require removal, many of them specify that the land rights will terminate when the line is no longer in use, or they specify that the public entity may require the line to be removed at any time.  For these agreements, I assumed that there will be a negotiation process that will take place between Northwest and the public entities, and that Northwest will be able to negotiate one of two results:  the line will be abandoned in place and filled with nitrogen, or the line will be abandoned in place and filled with a grout material.  I calculated the costs assuming half of the lines covered by these agreements will be filled with nitrogen, and half will be filled with the grout.  I have assumed that notwithstanding the fact that some public easements contain language requiring outright removal of the pipeline, the less disruptive approach of abandoning the transmission pipeline in place will be used for public lands.

Q.	How did you account for highway, road and canal crossings?

A.	In order to mitigate the possible effects of future collapsed pipelines, I expect that all highway, road and canal crossings, with pipe diameters of 10 inches or larger, will have to be isolated and filled with grout.  All railroad crossings, regardless of pipe size will also be filled with grout.

Q.	How did you treat the pipeline sections that cross agricultural lands?

A.	For agricultural lands, I looked at two options:  removal of the pipe where deep plowing or land leveling is needed (primarily affecting irrigated lands), or filling the pipe with cellular grout.  Northwest will not likely be allowed to abandon in place with a nitrogen fill in agriculture areas because of the risk of the ultimate collapse of pipe.  By reviewing the system alignment sheets, I determined that 231.5 miles of the pipeline cross irrigated fields, and I have assumed that these sections will have to be removed.  For the rest of the pipe in agricultural areas, 687 miles, I have assumed the pipe will be abandoned in place and filled with cellular grout.

Q.	Do the easement agreements across agricultural lands require Northwest to remove the line or fill it with grout?

A.	No.  Generally the easement agreements are silent on the issue of what happens to the line upon abandonment.  However, in Columbia Gas Transmission, 87 FERC 61,019 (1999), the Commission ordered that where the easement agreement was silent on the issue of how the pipe would be handled upon abandonment, the pipeline company would have to determine if the landowners wanted the line removed.  Based on this precedent, I assumed that the Commission will likely require Northwest to negotiate with the landowners to determine how the abandonment will be handled.  I based my cost estimate on the assumption that, in most instances, absent some clear agricultural need for removing the pipe, Northwest will be able to negotiate to follow the less intrusive approach and leave the pipe in place rather than having to remove it.

Q.	Are there any other issues you addressed with regard to abandonment of the pipe?

A.	Yes.  Currently, the Northwest system has 524 miles of pipe covered with a coating that contains asbestos.  For purposes of this study, I assumed that a portion of the 524 miles would be removed based on the above-discussed easement agreement requirements and landowner preferences.  For the remaining lengths, I assumed that half will be removed or remediated between now and final abandonment.  Upon final abandonment, whatever asbestos-coated pipe still remains will have to be removed and disposed of in accordance with federal and state laws.

Also, in western Washington, specific environmental mitigation measures are required for Class 1, 2, and 3 stream crossings.  I used current environmental standards that were applied to Northwest’s Capacity Replacement Project to determine the requirements and associated costs for abandonment at these crossings.  The result of my assessment of these current agency requirements is that half of the crossings will be filled with grout, and half of the crossings will be subject to removal of the line and mitigation of the effects on the river banks.

Q.	How did you calculate compression facility abandonment costs?

A.	To determine the costs to remove compression facilities, I considered external industry resources and Northwest’s recent project experience which includes the removal of two complete compressor stations.  The removal of those stations provided the costs for future removal of compression facilities.  The costs and the salvage values associated with the abandonment of the compression facilities are reflected on Schedule 4 of the Study.

Q.	What salvage values did you add from the compressor stations?

A.	I included the operations district office buildings in this portion of the study.  Most of the offices are anticipated to have a commercial resale value.  Apart from the offices, all other compression materials will have only scrap material value.

Q.	How did you determine the costs of abandoning the measurement facilities?

A.	I used estimates from internal Northwest project management and external industry resources.  The costs for abandonment of the measurement facilities include the removal of most aboveground measurement facilities and abandonment in place of all belowground measurement facilities, except where easement agreements require removal of belowground facilities.  These costs are reflected on Schedule 5 of the Study.

Q.	Do you project any salvage value from the measurement facilities?

A.	Yes.  However, because the majority of the equipment will only have scrap material value, there will only be a small amount of value received.  The overall costs of abandonment are greater than the salvage value.

Q.	How did you account for abandonment of the pipeline communication system?

A.	As shown on Schedule 6, my study of the abandonment of the pipeline’s communication system resulted in an overall positive salvage.  Most of the facilities can be resold, and the resale value will be greater than the costs of abandoning those sites that will not have any resale value.  For rate recovery, communications costs are generally spread over all facilities, but for convenience and for the purposes of this study only, I have applied the net credit against transmission salvage costs.

Q.	Are there any other facilities or assets that have a positive salvage?

A.	Yes.  The salvage value for the line pack gas is positive.  My study assumes that the existing compression will be used to pull the pressure in the lines down to 400 pounds.  A contractor will then bring in portable compression to remove the remaining gas, up to 94 percent of the total line pack.  The remaining six percent will be blown down and not recovered.  I valued the recoverable line pack gas using the midpoint averages from Platts Gas Daily Price Guide for the three years ended March 31, 2006.  This amount is $5.80 per dekatherm.  In addition to the positive value from the line pack gas, the salvage material from the aerial spans is also positive.

Q.	What are overheads?

A.	Overheads are costs that include general and administrative costs that are necessary to support a project but are not directly assigned to the project.  Consistent with Northwest’s typical overhead treatment, I have included a one percent factor to cover such costs.  As noted on Schedule 2, I have treated this small amount as a component of transmission.

Q.	How did you arrive at the salvage costs for underground storage (Jackson Prairie)?

A.	Abandonment of Jackson Prairie includes abandonment of the injection and withdrawal wells, removal of the gathering and transmission pipelines, removal of compression facilities and shop.  Northwest owns a one-third interest in Jackson Prairie; accordingly, my study includes only one-third of the total abandonment costs for the facility.  The costs and credits associated with the abandonment of Jackson Prairie are reflected on Schedule 7 of the Study.

Q.	What credits did you include for this facility?

A.	I included salvage value from the pipes and other scrap material associated with the storage facilities and from the sale of the office buildings.

Q.	Please discuss the portion of your study related to other storage facilities (Plymouth LNG).

A.	This portion of the study applies to the Plymouth LNG Plant.  The study covers the costs for dismantling two LNG tanks and other aboveground facilities.  The credit for the sale of the tanks and other equipment and material was also estimated using external industry resources.  These costs and credits are reflected on Schedule 8 of the Study.

Q.	What are the final net negative salvage values shown in your study?

A.	As detailed on Schedule 2, the cost of removal is $606,261,464, less the salvage I expect to recover, giving a total net negative salvage of $541,648,746.

Q.	Does this conclude your testimony?

A.	Yes it does.