UNITED STATES OF AMERICA

BEFORE THE

FEDERAL ENERGY REGULATORY COMMISSION

Nevada Power Company	)

and	)

Sierra Pacific Power Company	)

and    	)

Portland General Electric Company          )	Docket No. EC00-___-000

Prepared Direct Testimony of

Charles A. Pottey

- Please state your name and address.

A. 	My name is Charles A. Pottey. My business address is Sierra Pacific Power Company, 6100 Neil Road, PO Box 10100, Reno, Nevada 89520-0024.

Q.	By whom are you employed, and what is your position?

A. 	I am employed by Sierra Pacific Power Company (“Sierra Pacific” or “Sierra”) as a Senior Engineer in the Resource Planning Department

Q.  	DOES EXHIBIT ___ ACCURATELY REFLECT your educational background and professional experience.

A.  	Yes, it does.

Q.	what is the purpose of your testimony in this proceeding?

A.	In order to conduct his delivered price analysis, Dr. Fox-Penner requires an estimate of the market price in the Sierra Pacific control area.  As a proxy for the market price within the Sierra Pacific system, Dr. Fox-Penner has asked me to calculate the system lambda. The purpose of my testimony is to explain how the system lambda was calculated for Sierra Pacific Power Company.

Q.	PLEASE EXPLAIN WHAT sYSTEM lAMBDA IS?

A.	System Lambda is the incremental cost that the utility incurs to serve the next increment of load. It is the lower of the incremental cost of generation or the incremental cost of purchase power. System constraints such as available generation capacity and system import limits have to be included in this evaluation.

Q.	DOES SIERRA PACIFIC MAINTAIN A RECORD OF THE SYSTEM LAMBDA COST?

A.	Sierra Pacific maintains records of the incremental system cost based on the cost of the next increment of generation. However, these costs do not include the incremental cost of additional purchase power.

Q.	WHY ISN’T THE INCREMENTAL COST OF PURCHASE POWER INCLUDED IN THE INCREMENTAL SYSTEM COST DATA?

A.	The incremental cost of purchase power was not included in the incremental system cost data because Sierra Pacific does not have records of the hourly incremental purchase power cost available. The reason for this is that purchase power is not usually purchased at an hourly incremental cost. Instead it is purchased in blocks over a number of hours, days, weeks, or months. The purchases are usually made at least the day ahead and in some cases weeks or months ahead. The cost of purchase power in any given hour is a combination of many different purchases of varying duration and cost. The cost of a block of power that was purchased at the same cost for every hour in a month is a poor indication of what the incremental cost of additional purchase power would have been in any single hour.

Q.	DOES SIERRA PACIFIC MAINTAIN RECORDS OF THESE PURCHASES?

A.	Yes, Sierra Pacific maintains detailed records of all of its power purchases. Exhibit ___ is a copy of the WEIGHTED AVERAGE ENERGY COST REPORT for Tuesday, January 18, 2000. This shows an example of Sierra’s daily power purchases for a typical day. As can be seen it would be extremely difficult if not impossible to determine the incremental cost of an additional purchase for a specific hour from this data.

Q.	SINCE HISTORICAL HOURLY DATA FOR THE INCREMENTAL COST OF ADDITIONAL PURCHASE POWER IS NOT AVAILABLE FOR SIERRA PACIFIC’S SYSTEM, WHAT DATA WAS USED IN THE SYSTEM LAMBDA CALCULATION?

A.	The incremental system cost for additional purchased power delivered to Sierra Pacific’s system was developed from hourly data for the market price at appropriate trading hubs. The prices at these hubs were adjusted to include a basis differential to arrive at a delivered cost for Sierra Pacific’s system. The delivered cost of power was modeled as the Mid-Columbia index plus $2.25/MWH during all on-peak hours except for the months of July and August when the price is the California-Oregon Border (“COB”) index plus $5.00/MWH. During all off-peak hours the market price is the Mid-Columbia index plus $1.50/MWH. The higher basis differential during on peak hours is based on Sierra’s experience and is due primarily to higher transmission wheeling rates during on peak hours. The COB index is used in July and August because there are frequently transmission limitations in Idaho Power Company’s system during the peak load summer months limiting Sierra access to the Mid-Columbia hub. The hourly California-Oregon Border (“COB”) prices are based on the California Power Exchange (“PX”) day-ahead hourly market clearing prices.  The Mid-Columbia prices were produced by taking historical Power Markets Week daily on-peak and off-peak contract prices and shaping them based on the hourly price profile of the COB prices from the California PX.

Q.	HOW WAS THE SYSTEM LAMBDA COST CALCULATED AFTER THE INCREMENTAL COST FOR ADDITIONAL PURCHASE POWER WAS DETERMINED?

A.	The hourly incremental cost for additional purchase power was compared to the hourly incremental generation cost. If the incremental generation cost is lower than the delivered cost for additional purchases and additional generation capacity is available, then the incremental generation cost was used for the system lambda cost for that hour. If additional purchases cost less than additional generation and additional system import capacity is available, then the cost of additional purchases was used for the system lambda cost for that hour. Although Dr. Fox-Penner relies on market prices from the second quarter of 1998 through the end of the first quarter of 1999, I have calculated the system lambda for calendar year 1999.  This inconsistency exists because I felt that it was important to reflect the significant change in our ability to access short-term power markets due to the addition of the Alturas transmission line. This line was placed in service at the end of 1998 and resulted in an 83 percent increase in Sierra’s system import limit.

Q.	HOW WAS IT DETERMINED IF ADDITIONAL GENERATION CAPACITY WAS AVAILABLE?

A.	To determine if additional generation capacity was available, the available generation capacity account from the energy accounting system was compared to the system load account. If the generation capacity is greater than the system load, then additional generation is available. The incremental generation cost would then be used for the system lambda cost, if it is lower than the purchase power cost. If the load is greater than the generation capacity, then the delivered cost for additional purchases would have to be used for the system lambda even though it is higher than the incremental generation cost.

Q.	ARE THERE ARE PERIODS WHEN SIERRA’S LOAD IS GREATER THAN THE AVAILABLE GENERATION CAPACITY BUT THE GENERATION IS NOT FULLY LOADED?

A.	There are many periods when Sierra’s load exceeds available generation and the generation is not fully used. The unused generation is reserved for spinning and operating reserve. The only reason that the generation is not fully loaded is because Sierra Pacific has purchased external power, frequently at a cost that is higher than the internal generation cost, to reduce the loading on the generation. The appropriate cost to use for system lambda in this case is the external purchase power cost since this is the cost that would have to be incurred to maintain the required generation reserve margins.

Q.	WHEN additional purchases cost less than additional generation HOW DO YOU DETERMINE IF additional system import capacity is available?

A.	To verify if additional 	transmission import capacity is available, the net scheduled interchange for Sierra Pacific’s control area was compared to the system import limit. Sierra Pacific system import limit varies somewhat depending on system conditions but is generally around 660 MW. For this analysis it was assumed that for hours when the scheduled interchange is below 600 MW additional import capacity was available and, if the load had been higher, additional purchases could have been scheduled. If additional import capacity is available, the system lambda cost would be the delivered cost for additional purchases.  If additional import capacity were not available, the incremental cost for internal generation would be used for the system lambda cost.

Q.	HOW WAS THE SYSTEM LAMBDA MODELED DURING

PERIODS WHEN THE DATA INDICATED THAT NEITHER IMPORTS NOR GENERATION ARE AVAILABLE?

- There are 288 hours during 1999 when the data indicated that neither imports nor generation were available. This usually occurs when one of the generating units is out of service. During these periods, the incremental cost of additional purchases was used for the system lambda. For this analysis it was assumed that the transmission system was fully loaded when system imports reached 600 MW. Sierra’s system import limit is actually 660 MW. If there was not adequate generation or import capacity the margin between 600 MW and 660 MW would be utilized to serve this load. If necessary the system import limit would be exceeded for a short period of time to prevent interruption of load until additional generation resources were available.

Q.	WHAT WERE THE RESULTS OF THE SYSTEM LAMBDA CALCULATION?

A.	The hourly system lambda from the above calculation are included in Dr. Fox-Penner Workpaper 6. The average system lambda cost for the Sierra Pacific system for 1999 is $25.31/MWH.

Q.	DOES THAT CONCLUDE YOUR PREPARED DIRECT TESTIMONY?

A.	Yes, it does.

These hourly prices were provided to me by Dr. Fox-Penner, and the calculation is described in his Workpaper 6.