16868 - SEATTLE CITY LIGHT

Description of Economic Dispatch

Currently, Seattle City Light (Seattle) does not use a “classic” lambda formula to dispatch its hydroelectric resources to maximum efficiency..  The Seattle Energy Management System uses an arbitrarily set “water worth value” to derive a system lambda which feeds the Economic Dispatch function.  The water worth values are set to essentially place more value on the water of an upstream dam than a downstream dam as the water can be passed through multiple generators. The Economic Dispatch function uses generator performance curves to allocate generation in the most economic manner for those units that are in the economic dispatch mode.  As Seattle’s generators and plants on the Skagit project are not hydraulically matched and there are seasonal flow constraints for fish protection, not all units can be operated in the economic dispatch mode. Seattle normally strives to fully utilize its hydro storage and peaking capability at both its Boundary and Skagit Projects.  During the light load hours, we normally take in lower priced energy into our system, operate our Skagit Project to near minimum license requirements (fish flow minimum), and completely shut down our Boundary Project. During the heavy load hours, Boundary's entire daily inflow is generated only during those sixteen heavy load hours and not the entire day

Updated 5/14/01