Annual Control Area and Electric System Report

For the Year Ending December 31, 2002

Part II - Schedule 6.  Control Area System Lambda Data

Description of Economic Dispatch - System Lambda

Duke’s approach to economic dispatch involves the iterative process of evaluating system incremental costs, unit generation levels, and system load requirements.  The theory behind economic dispatch is to operate the generation units at an equal incremental cost (adjusted for transmission losses) to minimize system costs.  The process is conceptually described as follows:

- Select a system incremental cost to evaluate based on system load requirements.

- Use the selected system incremental cost to calculate unit incremental costs using unit penalty factors.  Unit penalty factors relate unit generation levels to system transmission losses.

- Calculate unit generation levels using the unit incremental cost and unit incremental cost curves.

- Sum up unit generation levels to get system generation.

- Compare system generation to system load requirements.

- If system generation is less than system load requirements, select a new system incremental cost that is greater than the previous system incremental cost.  If system generation is greater than system load requirement, select a new system incremental cost that is less than the previous system incremental cost.

- Return to step 2 and continue the process.

Once system generation equals the system load requirements (within a small deadband), the economic dispatch process is complete.  The last system incremental cost is system lambda, and the last unit generation levels are the optimal generation levels.