Demand and Supply in System Lambda Calculation

Demand: metered_gen + metered_imp + ru_req,

Where metered_gen = sum of metered reading values from generator in state.

( res_type = ‘GENERATOR’ and channel_number = 4),

metered_imp = sum of net metered read values from imports

(res_type  = ‘INTERTIE’ and

channel_number = 4 –channel_number = 1, i.e. imp-exp)

ru_req = regulation up requirement.

Suppply consists with following components:

non_thermal_net_out

mw:  sum of metered reading values from non thermal generator in state.

( res_type = ‘GENERATOR’ and channel_number = 4),

p:      price = 0.

metered_imp

sum of net metered read values from imports

(res_type  = ‘INTERTIE’ and

channel_number = 4 –channel_number = 1, i.e. imp-exp)

p:      price = 0.

thermal bid

mw: p_max – (scheduled outage + forced outage factor * p_max).

p:    average heat rate at p_max/1000 * zonal specific daily gas_price with

distribution charges + VAR_O_M.

import_adj_bid. DA import schedule adjustment anchored at mw with

weighted energy price.