FERC Form No 714

Year 2004

Part II - Schedule 6. Control Area System Lambda Data

Description of Economic Dispatch

CILCO’s hourly system lambda's for calendar year 2004 are stored as an ASCII file on the attached 3.5 inch diskette.  The file name is CILCOSL2004.TXT.   The file consists of 8760 records, which begin with the system lambda value for hour ending 1 AM, January 1, 2004.  The data was recorded in the central time zone.  A "NA" value was used for hour ending 3 a.m., April 4, 2004 due to the start of daylight savings time.  Only 24 hours of data were provided when daylight savings time ended at 2 a.m. October 31, 2004.  The system lambda units are dollars per megawatt hour.

The hourly system lambda is highest cost resource in a given hour.  It can be a generation resource or purchase power.  Data items used in the calculation of lambda are hourly system load, each unit’s average MW output for the hour, each unit’s fuel costs, each unit’s dispatch constants and purchase power costs.

The basic calculation is performed as follows.

- Calculate each units incremental cost using unit output, fuel cost and dispatch constants.

- If more than one interchange purchase is made in the hour, select the highest price purchase and discard the others.

- Choose the cost of the highest incremental cost resource for that hour as the system lambda for that hour.

The selection process just mentioned may be altered as follows.

Units on minimum load are excluded from setting the Lambda.  The minimum load for each unit is calculated as the highest of the unit rated minimum load times a factor (the current factor is 1.07), or the minimum unit output for that day multiplied by that same factor.  The factor is meant to account for operating conditions which might have the unit operating with a higher than normal minimum load.  In hours when all units are at minimum load, the lowest incremental cost resource sets the Lambda for that hour.

For the Sterling CTG units, the unit maximum of 16 MW is used in calculation of incremental cost whenever the Sterling units show generation for an hour, even if the generation is less than 16 MW.  This is because these units typically run at full load when they are on, but the technique used for hourly integration of load data may cause the hourly generation level to be recorded at less than 16 MW.  However, if the Sterling unit generation shows up at less than 9 MW, that unit is excluded from setting the hourly Lambda.  The 9 MW value is indicative that the unit was on for significantly less than an hour and thus should not be used to set the lambda.

In an hour when the sum of net generation of coal fired units is greater than that hour’s load, purchases and CTG generation are excluded from setting the Lambda.  In such an hour, purchases and CTG’s are likely on for reasons such as minimum up time, unit testing, or satisfaction of a purchase agreement.  These resources would not be properly considered dispatchable marginal resources under such conditions.