TRANWESTERN PIPELINE COMPANY, LLC

Description of System Design and Operation

SYSTEM DESIGN

Transwestern Pipeline Company, LLC (“Transwestern”) designs its transmission system on the basis of simultaneous demand for all maximum contractual firm obligations.  Compression and pipe facilities are designed on the basis of maximum daily flow rates.

For pipeline system design purposes, Tranwestern utilizes WinFlow Version 4.06, steady state, Natural Gas Pipeline Simulation for Windows from Gregg Engineering.  Additionally, Transwestern on a case by case basis will utilize WinTran Version 4.06, Transient Natural Gas Pipeline Simulation for Windows, from Gregg Engineering.  Calculations within these applications are based on flow equations and assumptions commonly accepted in the interstate gas pipeline industry.  The models incorporate data from gas compositional analysis and actual pipeline flow tests to ensure the accuracy of the calculated flow results.

Various equations and subroutines are used by the simulation model to calculate pressures along the system.  A description of the major basic equations and the variables used in these equations is set forth below:

Pipeline Flow Equation

Where:

Q			=	Volume of gas flowing in millions of standard cubic feet per day

P1			=	Upstream pressure, psia

P2			=	Downstream pressure, psia

Pavg		=	Average pressure between P1 and P2, psia

D			=	Inside pipe diameter, inches

L			=	Length of pipe, miles

Pb			=	Base pressure, 14.73 psia

Tb			=	Base temperature, 520 degrees Rankin

Tavg		=	Average flowing temperature, degrees Rankin

G			=	Specific gravity of gas (Air = 1.0)

h1			=	Upstream elevation, feet

h2			=	Downstream elevation, feet

zavg		=	Average gas compressibility in pipeline segment

f			=	Friction factor from Colebrook-White equation.

The Colebrook-White equation is given by:

Where:

e		=	Pipe roughness, inches

Compressor Horsepower Equation

Where:

HP		=		Compressor horsepower

r		=		Compression ratio,   Pd

¾¾

Ps

Ps		=		Suction pressure, psia

Pd		=		Discharge pressure, psia

Ts		=		Suction temperature, degrees Rankin

k		=		Ratio of specific heat,        Cp

¾¾   =	1.286

Cv

E		=		Compressor efficiency

Zs		=		Compressibility factor at compressor suction pressure and 								temperature

In addition to these two equations, the program determines the flowing temperature by calculating the temperature drop along the pipeline based on ground temperature and the thermal conductivity of the pipe.  The compressibility is also calculated using the average pressure and the flowing temperature in the segment.  Compressor discharge temperature is calculated based on the compression ratio and gas properties.

Compressor station suction and discharge piping pressure loss is estimated at a maximum of 5 psi.  Additionally, natural gas fueled compressor station horsepower is de-rated to reflect the effects of elevation and ambient temperatures at station locations.

The Transwestern system capacity is determined by the capability of the pipeline and compression facilities of each segment.  This capacity varies with the ground temperature which swings seasonally.  Since this temperature variation is accompanied by a minor increase in the pipeline capacity during the winter, design capacities are based on summer conditions only.

Transwestern’s system is designed to meet all firm transportation contract obligations at any time.  Since the pipeline capacity is reduced when the flowing temperature is increased, calculations were based on summer ground temperatures.  The average flowing gas temperature was calculated within the flow model using ground temperature and thermal conductivity.

The pipeline roughness will remain fairly constant when the gas stream does not include any free liquid; however, due to the changes in temperature and pressure which occur along the various systems coupled with the composition of the gas, the "effective" pipe roughness does change with time due to condensation of the liquid hydrocarbons.  The pipeline roughness for Transwestern’s mainline system ranges between 500 and 700 micro-inches.

The gas properties of the existing system are well documented; however, the gas supply to meet the needs of the existing facilities varies daily.  Therefore, the following values, which are judged to be representative, are used for both existing and proposed facilities.

| Gas Property | Value |
| --- | --- |
| Gravity | Range 0.570 – 0.600 (Air = 1) |
| Ratio of specific heat | Calculated as needed by the model |
| Compressibility factor | Calculated as needed by the model |

SYSTEM OPERATION

Transwestern’s system receives gas from four primary supply basins, Anadarko, Permian, Rockies and San Juan.  Transwestern receives this gas for transport to a number of delivery markets including New Mexico, Texas, Arizona, Nevada, and California.  Although a significant portion of Transwestern’s system is bi-directional, gas flow is predominantly out of the San Juan Basin moving southward to Thoreau, NM (terminus of the San Juan Lateral) then west and eastward towards California and Texas, respectively.  Less significant, but still noteworthy, is the volumetric flow from the Anadarko and Permian Basins.

All facilities on Transwestern’s system east of Thoreau, New Mexico are considered bi-directional.  These bi-directional facilities are more easily identifiable as the Panhandle Lateral, West Texas Lateral, and Mainline East segments of the system.

- Panhandle Lateral - The “Panhandle Lateral” is defined by three compressor stations (designated P-1, P-2, and P-3) and approximately 290 miles of single pipe from Station No. 9 (Roswell, NM) to its terminus in Hemphill County, TX.  Typically, gas will flow in a northeast direction moving San Juan and Permian Basin gas to the Mid-Continent inter-state pipelines; however the capability does exist for gas to move in a southwest direction moving gas from the Anadarko Basin towards Station No. 9.

- West Texas Lateral - The “West Texas Lateral” is defined by two major compressor stations (designated WT-1 and WT-2), several booster compressor stations and approximately 175 miles of mostly looped pipe from Station No. 9 to an area near Monahans, TX.  Typically gas will flow in a southeast direction, moving San Juan and Permian gas to Texas intra-state pipelines near the Waha Hub. However the capability does exist to move gas in a northwest direction, from the Permian Basin (Waha Hub) towards Station No. 9.

- Mainline East - The “Mainline East” facilities are defined by five compressor stations (designated Station Nos. 5-9) and approximately 285 miles of partially looped pipe from Thoreau, NM (at the terminus of the San Juan Lateral) to Station No. 9.  Gas will typically flow in an eastward direction, from the San Juan Basin towards the mid-continent and Waha Hub markets; however, the capability does exist to move gas in a westward direction from Permian and Anadarko Basins to Arizona, Nevada and California markets.

Transwestern’s shippers have demonstrated the most significant supply area to be the San Juan Basin.  Transwestern’s “San Juan Lateral” is a partially looped 130 mile lateral extending from La Plata County, Colorado to its mainline terminus at Thoreau, NM.  The San Juan Lateral transports not only San Juan Basin gas from the Blanco Hub in Bloomfield, NM but is also a significant avenue for Rockies production (via Northwest Pipeline and TransColorado).  Once San Juan Lateral gas is received into the mainline at Thoreau, NM, gas is typically transported both eastward (towards Station No. 9) and westward (Arizona, Nevada and California Markets).

The “Mainline West” segment of Transwestern’s system is defined by four compressor stations (designated Station Nos. 1-4) and approximately 360 miles of looped pipe from Thoreau, NM to the Colorado River (California/Arizona border).  Most of the gas transported on the Mainline West is sourced from San Juan and Rockies production; however, Permian and Anadarko gas (by displacement) is also contracted westward.