Exercise 1.11, RMC-BestFit

Flow Intervals and Perception Thresholds

Disclaimer: The United States Army Corps of Engineers has granted access to the information in this hands-on exercise for instructional purposes only.

References:

- RMC-TR-2019-07: Data Sources for Estimating Hydrologic Hazards for Semi-Quantitative Risk Assessments (https://www.rmc.usace.army.mil/Library/RMC-Publications/)

- RMC-TR-2020-03: RMC-BestFit Quick Start Guide (https://www.rmc.usace.army.mil/Library/RMC-Publications/)

- Bulletin 17C: Guidelines for Determining Flood Flow Frequency (https://pubs.usgs.gov/tm/04/b05/tm4b5.pdf)

Background:

You have been tasked with improving the flood hazard analysis for Example Dam using available historical and paleoflood information.

It is important to collect as much historical and paleoflood at-site information as practical when performing a flood hazard analysis. Common data sources include USGS published gage records, water supply papers, design documents, operation manuals, flood reports, newspaper accounts, and paleoflood studies.

Required Software:

- RMC-BestFit

- Microsoft Excel

Primary Objectives:

The following tasks will be performed during this hands-on exercise:

- Research historical flood information

- Estimate volumes and corresponding flow intervals for historical floods

- Estimate historical periods and corresponding perception thresholds

- Calculate a flow frequency curve given historical information

Additional Objectives:

If time allows complete the following additional tasks:

- Calculate a flow frequency curve given paleoflood information

- Compare frequency curves

Products:

At the end of this exercise, you will have a flow frequency curve based on the available at-site systematic, historical, and paleoflood information.

Files Needed for Exercise:

|  | File Name | File Type | Description |
| --- | --- | --- | --- |
| 1 | E1.11.xlsx | Microsoft Excel Spreadsheet | Peak to Volume |
| 2 | E1.11.bestfit | RMC-BestFit Model | Flow Interval and Perception Threshold Data Model |

Task 1.  Research historical flood information

- The screenshot below summarizes historical flood information that was obtained for a discontinued USGS gage located just downstream of Example Dam. The information suggests that two historical floods were recorded prior to the systematic record. These floods occurred in water years 1898 and 1910 with peak flow estimates of 94,000 cfs and 21,500 cfs.

- Question: What is your recommendation for including or excluding these historical floods for the flood hazard analysis for Example Dam?

Answer:

- Include both historical floods

- Include the 1898 flood only

- Include the 1910 flood only

- Include neither historical flood

Task 2.  Estimate volumes and flow intervals for historical floods

- Since only peak flow estimates are available for the historical floods, volumes need to be estimated based on other available information. This exercise assumes the critical inflow duration for Example Dam is 4-days. Therefore, a 4-day volume needs to be estimated for the historical floods.

- One method is to estimate a typical peak to volume ratio based on several observed floods. A nearby discontinued USGS gage has published peak and daily flow data for water years 1922 through 1958 that can be used. The drainage area at the gage is 1238 sq. mi. and the drainage area at Example Dam is 1186 sq. mi. The difference in drainage area is assumed to be negligible for this exercise since the uncertainty in the 4-day volume estimate is likely to be much larger than the difference due to drainage area. Three floods having a peak flow like the flood of 1898 and three floods having a peak flow like the flood of 1910 have been selected to estimate a peak to volume ratio.

- Find the annual maximum peak flow for the three floods listed in the table below from the downstream gage historical flood information above and enter the peak flow values in the table. Launch the RMC-BestFit software and open the E1.11.bestfit file, open the “DS_USGS_Daily_Inflow” Time Series Data.  Notice there is systematic daily data from October 1921 to September 1966, which covers the three historic events in the table. In the BestFit file, create a new Input Data titled “DS_USGS_4-Day”, where we will calculated the annual maximum 4-day average inflows. In the properties window, select the following Exact Data Options, then click process.

- You have now calculated the maximum 4-day volumes for each water year. Enter the maximum value in the table below for each associated year and month of flood.

Calculate a peak to volume ratio for the three floods and enter the values in the table below.

| Flood | Water Year | Peak Flow (cfs) | Maximum 4-Day Volume (cfs) | Ratio of Peak to Volume |
| --- | --- | --- | --- | --- |
| April 1927 | 1927 |  |  |  |
| May 1943 | 1943 |  |  |  |
| April 1945 | 1945 |  |  |  |

- Question: Based on these three observed floods, what is your recommended peak flow to 4-day volume ratio for floods having a peak flow like the flood of 1898?

- Answer:

Ratio:

- Question: Based on your estimate of the peak to volume ratio what is your estimate of the 4-day volume for the flood of 1898?

- Answer:

Most Likely 4-Day Volume for Flood of 1898:

- Question: Assuming the uncertainty is about +/- 20%, what is your recommendation for the upper and lower 4-day volume estimates for the flood of 1898?

- Answer:

Upper 4-Day Volume for Flood of 1898:

Lower 4-Day Volume for Flood of 1898:

- Similar calculations have been performed for the flood of 1910, which has a recommended peak to volume ratio: 2.0

Most Likely 4-Day Volume for Flood of 1910: 21,500 / 2 = 10,750 cfs

Upper 4-Day Volume for Flood of 1910: 21,500*1.2 / 2 = 12,900 cfs

Lower 4-Day Volume for Flood of 1910: 21,500 * 0.8 / 2 = 8,600 cfs

- Similar calculations have been performed for the three floods having a peak flow like the flood of 1910.  Results are summarized in the table below.

| Flood | Water Year | Peak Flow (cfs) | 4-Day Volume (cfs) | Ratio of Peak to Volume |
| --- | --- | --- | --- | --- |
| May 1924 | 1924 | 23,500 | 12,218 | 1.9 |
| April 1939 | 1939 | 19,700 | 9,145 | 2.2 |
| April 1952 | 1952 | 24,100 | 12,595 | 1.9 |

- Task 3.  Estimate historical periods and corresponding perception thresholds

- The following quotations were found in the original design report for Example Dam and were corroborated by similar accounts found in several newspaper archives.

- “The great flood of 1898 is one of the greatest floods on record and the largest known flood to have occurred prior to the flood of 1943.”

“The flood of 1898 apparently was one of the greatest floods on record for the Example River since settlers first arrived at Yourhome Towne circa 1865. Notable floods that were smaller than the flood of 1898 occurred in 1873 and 1886, but discharge estimates are not available for these floods.”

- Question: Given the available historical flood information, what is your recommended historical period for Example Dam?

- Answer:

- 1898 – 1943

- 1865 – 2017

- 1865 – 1921

- 1873 – 1921

- Question: Given the available historical flood information, what is your recommended perception threshold for the historical period?

- Recall that a perception threshold means that unobserved floods that occurred during the historical period are likely to have been less than the perception threshold value based on the available evidence. Since the flood hazard analysis is based on a 4-day critical duration, the perception threshold should be based on a 4-day volume.

- Answer:

- Perception Threshold:

- Task 4.  Calculate a flow frequency curve given historical information

- Launch the RMC-BestFit software and open the E1.11.bestfit project.

- In the E1.11.bestfit project open the Historical input dataset by right-clicking on the name of the input dataset (Historical) and selecting Edit. Navigate to the Interval Data tab and insert one or more new rows by selecting the Add Rows(s) to the Bottom of the Table icon. The number of rows inserted should be based on the number of historical floods you have decided to include in the flood hazard analysis. Recall the two historical flood candidates are the floods of 1898 and 1910.

- Enter the Year, Lower, Most Likely, and Upper 4-day volume estimates for each historical flood(s) you have decided to include in the flood hazard analysis. The screenshot below shows a possible entry for the flood of 1898. Your specific entries and values could be different depending on which historical floods you decided to include and your estimate for the peak to volume ratio.

- Navigate to the Threshold Data tab which is located next to the Interval Data tab. Insert a new row by selecting the Add Rows(s) to the Bottom of the Table icon.

- Enter the Start Year, End Year, and 4-day volume for the perception threshold. Recall that perception thresholds should typically not overlap historical flood events. The screenshot below shows a possible entry for a perception threshold. Your specific entries and values could be different depending on which historical floods you decided to include and what historical period you selected.

- Open the Historical Bayesian Univariate Distribution Analysis by double left-clicking on the name of the analysis (Historical).

- Run the analysis by clicking on the Estimate button on the Properties frame. When the analysis is complete, explore the results by selecting the Frequency Plot, Tabular Results, and Parameter Sets tabs.

- Question: What are the parameter estimates for the posterior mean? These can be found in the Posterior Mean Summary Statistics table on the Tabular Results tab.

Answer:

Mean:

Standard Deviation:

Skew:

Task 5. Calculate a flood frequency curve given paleoflood information

- A paleoflood study has been completed for Example Dam. The study identified one paleoflood stage indicator (PSI) based on geologic evidence of a pre-historic flood. The results of the paleoflood study are summarized in the table below. Recall that standard practice is to use 1 January 1950 as the commencement of the age scale for years before present. The 4-day volume estimates are based on a peak to volume ratio of 1.8. The uncertainty in the flow and volume estimates is based on geologic uncertainty in the elevation of the flood and hydraulic modeling uncertainty.  The uncertainty in the age estimate is based on uncertainty in the carbon dating results.

|  | Age in Years Before Present | Peak Flow Estimate for PSI (cfs) | 4-Day Volume (cfs) |
| --- | --- | --- | --- |
| Lower | 700 | 182,000 | 101,000 |
| Most Likely | 850 | 260,000 | 144,000 |
| Upper | 1000 | 338,000 | 188,000 |

- Question: Given the paleoflood age information, what is your recommendation for the start year of the paleoflood historical period?

Answer:

- 1167

- 1250

- 1100

- 950

- Question: What is your recommendation for the water year of the PSI. Recall that the specific year you choose is not critical so long as the paleoflood stage indicator (PSI) is included in the pre-historic period. A typical practice is to assume the flood associated with the PSI occurred at the beginning of the pre-historic period.

Answer:

- Water Year for PSI:

- Return to RMC-BestFit and the E1.11.bestfit project. Copy the Historical input dataset to a new input dataset. This can be done by right-clicking on the name of the Historical input dataset in the Project Explorer frame and selecting Copy… Name this new input dataset Paleoflood.

- Open the new Paleoflood input dataset and navigate to the Interval Data tab. Insert a new row in the table. Enter the Year, Lower, Most Likely, and Upper 4-day volume estimates for the paleoflood stage indicator (PSI).

- Navigate to the Threshold data tab. Select the first row in the table and insert a new row at the top of the table by clicking the Insert Row(s) Into the Table icon. Inserting a new row at the top of the table will ensure that the perception thresholds are in chronological order. Enter the Start Year, End Year, and Value (4-day volume) for the perception threshold associated with the paleoflood stage indicator (PSI).

- Copy the Historical Bayesian Univariate Distribution Analysis to a new analysis. This can be done by right-clicking of the analysis name (Historical) and selecting copy. Name this new analysis Paleoflood.

- Select the Paleoflood input data from the drop-down menu on the Properties window. Run the new analysis by clicking the Estimate button on the Properties window.

- When the analysis is complete, explore the results by selecting the Frequency Plot, Tabular Results, and Parameter Sets tabs.

- Question: What are the parameter estimates for the posterior mode? These can be found in the Posterior Mode Summary Statistics table on the Tabular Results tab. Recall that the posterior mode is like a computed flow frequency curve.

- Answer:

Mean:

Standard Deviation:

Skew:

Task 6. Compare the frequency curves

- Visually explore and compare the results for the Period of Record, Extended Period of Record, Historical, and Paleoflood Bayesian estimation analyses using the Graphical Results tab.

- Question: Does the paleoflood information appear to be consistent with the systematic and historical information?

- Answer:

- Question: Did the paleoflood information change the posterior predictive curve? Why or why not?

Answer:

- Question: Did the paleoflood information reduce the uncertainty? Why or why not?

Answer: