Exercise 1.14, RMC-BestFit

Distribution Fitting Analysis

Disclaimer: 	The United States Army Corps of Engineers has granted access to the information in this hands-on exercise for instructional purposes only.

References:

- RMC-TR-2020-03: RMC-BestFit Quick Start Guide (https://www.rmc.usace.army.mil/Library/RMC-Publications)

Background:

You have been tasked with exploring alternative distributions for the Example Dam flood hazard analysis.

Required Software:

- RMC-BestFit

Primary Objectives:

The following major tasks will serve as an outline for the hands-on exercise:

- Explore alternative three parameter distributions

Additional Objectives:

If time allows complete the following tasks for additional learning objectives:

- Explore distribution fitting analysis for alternative input datasets

- Compare distribution fitting analyses

Products:

At the end of this exercise, you will have a distribution fitting analysis that can inform selection of a distribution for the flood hazard analysis.

Files Needed for Exercise:

|  | File Name | File Type | Description |
| --- | --- | --- | --- |
| 1 | E1.14.bestfit | RMC-BestFit Model | Distribution Fitting Model |

Task 1.  Explore alternative three parameter distributions

- Log-Pearson Type III (LPIII) is the most used distribution in the United States for flood frequency analysis because it is consistent with the Bulletin 17C Guidelines for Determining Flood Flow Frequency. RMC-BestFit offers several other distributions and provides a distribution fitting analysis to evaluate the fit of various distributions to the data.

- The distribution fitting analysis can also be used as a tool to assess whether to exclude low-outliers from a flood hazard analysis based on the Multiple Grubbs-Beck Test (MGBT). Refer to Bulletin 17C (https://pubs.usgs.gov/tm/04/b05/tm4b5.pdf) for details on MGBT. A distribution fitting analysis can be performed with and without low-outliers to evaluate which option provides a better fit to the data.

- Launch the RMC-BestFit software and open the E1.14.bestfit Distribution Fitting project.

- Create a new distribution fitting analysis by right-clicking on Distribution Fitting Analysis in the Project Explorer frame or by selecting New Distribution Fitting Analysis from the Project menu. Name the new analysis Paleoflood.

- Select the Paleoflood input dataset from the drop-down menu in the Properties frame and run the distribution fitting analysis by clicking the Estimate button.

- RMC-BestFit allows the users to select the parameter distributions type by selecting the Type dropdown menu. Recall that 2-parameter distributions do not have a shape parameter. For most cases we will want to look at the 3-parameter distributions, therefore select the 3-parameter option. The screenshot below shows the result.

- Results can be sorted for a particular goodness of fit metric by left-clicking on the column heading in the table.

- Question: Which three parameter distribution is the best fit to the data based on the RMSE criteria? Recall that a smaller RMSE is better.

Answer:

Three Parameter Distribution Based on RMSE:

- Question: Which three parameter distribution is the best fit to the data based on the BIC criteria? Recall that a smaller BIC value is better.

Answer:

Three Parameter Distribution Based on BIC:

- Question: Which three parameter distribution is the best fit to the data based on the AIC criteria? Recall that a smaller AIC value is better.

Answer:

Three Parameter Distribution Based on AIC:

- Task 2.  Explore distribution fitting analysis for alternative input datasets

- Create a new distribution fitting analysis for one of the other input datatsets and name the new analysis using the same name as the input dataset.

- Select the input dataset using the Input Data drop-down menu in the Properties frame. Run the new Distribution Fitting Analysis by clicking the Estimate button.

- Question: Which three parameter distribution is the best fit to the dataset you selected based on the AIC criteria? Recall that a smaller AIC value is better.

Answer:

Dataset:

Three Parameter Distribution Based on AIC:

- Task 3.  Compare distribution fitting analyses

- Compare the results of the two distribution fitting analyses.

- Question: Was the same three-parameter distribution identified as having the best fit to each dataset based on all three criteria (AIC, BIC, RMSE)? Which distributions were identified as the best?

Answer:

- Question: What alternative three parameter distribution(s) might be a reasonable choice for this flood hazard analysis based on the distribution fitting analysis?

Answer: