Exercise 1.22, RMC-BestFit

Prior Information

Disclaimer: The United States Army Corps of Engineers has granted access to the information in this workshop for instructional purposes only.

References:

- RMC-TR-2020-03: RMC-BestFit Quick Start Guide (https://www.rmc.usace.army.mil/Library/RMC-Publications/)

- Regional Skew Studies by the USGS (https://www.usgs.gov/streamstats/science/flood-frequency-reports)

- NOAA Atlas 14 Point Precipitation Frequency Estimates (https://www.weather.gov/owp/hdsc)

Background:

You have been tasked with improving the flood hazard analysis for Example Dam. In this workshop, you will add regional skew information and information from a precipitation frequency study.

It is important to collect as much additional information as practical when performing a flood hazard analysis. Common options include a prior distribution on skew based on a regional skew study and a prior distribution on quantiles based on a precipitation frequency study.

Required Software:

- RMC-BestFit

Primary Objectives:

The following major tasks will be performed during this workshop:

- Calculate a flow frequency curve with regional skew information

- Calculate a flow frequency curve with precipitation frequency information

Additional Objectives:

If time allows complete the following additional tasks:

- Explore RMC-BestFit diagnostic tools

- Compare flow frequency curves using one versus three quantile priors

Products:

At the end of this workshop, you will have a flow frequency curve for Example Dam based on all available information.

Files Needed for Exercise:

|  | File Name | File Type | Description |
| --- | --- | --- | --- |
| 1 | E1.22.bestfit | RMC-BestFit Model | Prior Information Model |

Task 1.  Calculate a flow frequency curve with regional skew information

- A flood hazard study should include research on available regional skew studies and in some cases should consider performing a custom regional skew study. A recommended resource for modern regional skew studies is https://www.usgs.gov/streamstats/science/flood-frequency-reports. Recall that older regional skew estimates such as the skew map in Bulletin 17B are not recommended for use.

- The following screenshot summarizes information that was obtained from a modern regional skew study published by the USGS for the state where Example Dam is located.

- Question: Does the drainage area for Example Dam (1186 sq. mi.) fall within the range of drainage areas included in this regional skew study?

- Answer:

- The following screenshot summarizes additional information that was obtained from the regional skew study.

- Question: What value of regional skew and regional skew mean square error (MSE) are recommended by the regional skew study?

- Answer:

- Regional Skew:

- Regional Skew Mean Square Error:

- Question: Assuming the MSE of the regional skew is equal to the variance, what is the recommended standard deviation for the regional skew value? Recall that standard deviation is equal to the square root of the variance.

- Answer:

- Regional Skew Standard Deviation (Square Root of MSE):

- Regional skew estimates are entered in RMC-BestFit as a prior distribution on the skew parameter in a Univariate Distribution Analysis. Launch the RMC-BestFit software and open the E1.22.bestfit project. Copy the Paleoflood Univariate Distribution analysis by right-clicking on the name in the Project Explorer frame. Name the new analysis Regional Skew.

- In the new Regional Skew Univariate Distribution analysis, uncheck the Use Default Flat Priors box in the Properties window by clicking on the checkbox.

- Select the distribution for skew by left-clicking on the U(-2,2) cell next to the Skew parameter.

- Change the distribution type to Normal using the drop-down menu. Enter the regional skew value that you selected from the regional skew study as the mean value and enter the standard deviation that you calculated as the standard deviation value.

- Run the Regional Skew Univariate Distribution analysis by clicking the Estimate button.

- When the analysis is complete, explore the results by selecting the Distribution Results and Tabular Results tabs.

- Question: What are the parameter estimates for the posterior mean frequency curve? These can be found in the Posterior Mean Summary Statistics table. Recall that the posterior mean is the average of the parameters for the 10,000 posterior distributions.

- Answer:

- Mean:

- Standard Deviation:

- Skew:

- Task 2.  Calculate a flow frequency curve with precipitation frequency information

- A flood hazard study can include flow and volume estimates obtained from precipitation frequency estimates combined with a rainfall runoff model.

- The following figures and table summarizes 4-day inflow volume estimates for Example Dam based on NOAA Atlas 14 Point Precipitation Frequency Estimates. The point precipitation frequency estimates were converted into a set of representative storms based on estimated area reduction factors and observed precipitation patterns. The storms were modeled using a calibrated and validated HEC-HMS hydrologic model to obtain estimated inflow hydrographs at Example Dam. Maximum 4-day inflow volumes were then calculated from the modeled inflow hydrographs. Uncertainty was estimated by varying the point precipitation depths based on the NOAA Atlas 14 confidence intervals, variability in calibrated loss parameters in the hydrologic model, and variability in observed precipitation patterns. This method assumes that the AEP of the 4-day volume is approximately equal to the AEP of the point precipitation.

- Example Dam NA14 Precipitation-Frequency with 1000 realization bootstrap GEV Distribution

The figure below is a histogram and summary statistics of the NA14 precipitation-frequency at the 0.01 AEP or 0.01 quantile. This represents the 1000 realizations from the bootstrap GEV distribution of the precipitation-frequency. Notice this is mostly a normal distribution with a slight tail to the higher depths. Because often you will see a slight skew to the upper depths, RMC-BestFit 2 now defaults to Log-Normal (base e) distribution.

The figure below represents the 0.01 AEP or quantile flow distribution from a stochastic modeling process that samples the above precipitation-frequency depths and routes those through a hydrologic model. The mean and standard deviation can directly be entered into RMC-BestFit. If stochastic modeling isn’t possible, discrete runs that define the upper to lower possible distribution is adequate as well.

The table below summarizes the stochastic model of three quantiles 0.1, 0.01, and 0.001 AEP.

| Annual Exceedance Probability | 4-Day Volume |  |  |
| --- | --- | --- | --- |
|  | Reasonable Low (5%) Estimate (cfs) | Best Estimate (mean) (cfs) | Reasonable High (95%) Estimate (cfs) |
| 0.1 | 29,600 | 40,500 | 49,000 |
| 0.01 | 49,000 | 65,100 | 81,300 |
| 0.001 | 66,800 | 95,500 | 130,400 |

- Question: Which of the 4-day volume estimates from the precipitation frequency study do you recommend including in the flood hazard analysis?

- Answer:

- None

- 0.1

- 0.01

- 0.001

- All three

- Question: Assuming a log-normal distribution, what is your estimate of the mean and standard deviation for the 4-day volume estimates from the precipitation frequency study (table above)? Recall that the mean might typically be assumed to be equal to the best estimate and the standard deviation might typically be calculated assuming the range between the reasonable high and reasonable low estimates is about +/- 2 standard deviations. This is again the simplest approach. Most studies, whether creating discrete runs or using stochastic modeling, will calculate the mean and standard deviation from the sampled results.

- Answer:

- SD=High-Low4

| Annual Exceedance Probability | Mean | Standard Deviation |
| --- | --- | --- |
| 0.1 |  |  |
| 0.01 |  |  |
| 0.001 |  |  |

- Question: Do you expect the simple standard deviation to be more, less, or approximately the same as a stochastic result?

- Answer:

- Return to the RMC-BestFit software and the E1.22.bestfit project. Copy the Regional Skew Univariate Distribution analysis to a new analysis by right-clicking on the name in the Project Explorer frame and selecting Copy. Name the new analysis Precipitation Frequency.

- Check the Enable Priors on Quantiles box in the Properties frame by clicking on the checkbox. If you recommend including only one of the estimates, check the Use Single Quantile box. However, if you recommend including all three estimates in the flood hazard analysis, leave the single quantile box unchecked. The screenshot below is based on using three priors as this is standard.

- Enter the AEP for the estimate(s) that you have decided to include using the drop-down menu. Enter the distribution parameters for each estimate by left-clicking on the distribution cell. Make sure the Log-Normal (base e) distribution is selected using the drop-down menu and enter your estimate for the mean and standard deviation. The image below represents the stochastic modeling mean and standard deviation results for each quantile.

- Run the Precipitation Frequency Univariate Distribution analysis by clicking the Estimate button.

- When the analysis is complete, explore the results by selecting the Distribution Results andTabular Results tabs.

- Question: What are the parameter estimates for the posterior mean frequency curve? These can be found in the Posterior Mean Summary Statistics table. Recall that the posterior mean is the average of the parameters for the 10,000 posterior distributions.

- Answer:

- Mean:

- Standard Deviation:

- Skew:

- Task 3.  Explore RMC-BestFit diagnostic tools

- Up to this point you have spent most of the time in the Distribution Results tab. Take some time to explore some of the other outputs and diagnostic tools that are available in RMC-BestFit.

- The Kernel Density and Histogram tabs provide both a visual and numerical summary of the prior and posterior distributions for each parameter. Return to the RMC-BestFit software and the E1.22.bestfit project. Open the Regional Skew Univariate Distribution analysis and select either the Kernel Density or Histogram tab. Select the Skew parameter using the drop-down menu and select the Show Prior Distribution checkbox. Shown in blue is the prior distribution for skew which is the regional skew estimate.  Shown in red is the posterior distribution for skew which is like a weighted skew estimate. Summary statistics can be viewed by selecting the down arrow in the bottom left corner of the window.

- Question: Why is the prior distribution for skew wider than the posterior distribution for skew?

- Answer:

- The Bivariate tab provides a visual summary of the bivariate posterior probability density function for each pair of distribution parameters. Navigate to the Bivariate tab. Change the x parameter to standard deviation and change the y parameter to skew using the drop-down lists. Do you notice a diagonal pattern in the heat map?

- Question: Why does the bivariate distribution for standard deviation and skew typically have this diagonal pattern?

- Answer:

- The Mean Likelihood tab provides a visual check for model convergence. Navigate to the Mean Likelihood tab. Notice the mean likelihood changes significantly at the start of the evolutions and then becomes relatively stable. Stability in the mean likelihood is an indicator of good model convergence.

- The Autocorrelation tab provides a visual check of the correlation among the samples. The Markov Chain Monte Carlo (MCMC) samples are dependent, so there will be some correlation among consecutive samples. The samples need to be thinned by only using every kth sample to minimize this correlation. Navigate to the Autocorrelation tab. The plot shows the lag-k correlation between a sample and the sample from k steps before. Autocorrelation should generally decrease as k increases. The default thinning interval is set to k=30 which should provide practically zero correlation among the set of thinned samples.

- The Markov Chain Traces tab provides a visual check of the mixing of a Markov chain. Navigate to the Markov Chain Traces tab. Look for any anomaly in the trace plot. Are the chains relatively stable? Are any of the chains significantly different than the others? Do any of the chains jump from one range of parameter values to another? These can be signs of poor mixing and can indicate the need for more evolutions. Check the Include Warm-Up box. Notice the chains are unstable at the start of the evolutions until convergence is achieved.

- Task 4. Compare frequency curves using one versus three quantile priors

- Return to the RMC-BestFit software and the E1.22.bestfit project.

- Copy the Precipitation Frequency Univariate Distribution analysis to a new analysis by right-clicking on the name in the Project Explorer frame and selecting Copy. Name the new analysis Precipitation Frequency-Sensitivity.

- Previously the three precipitation frequency quantiles entered in Task 2. Now try running the analysis again by only including one precipitation frequency quantile (0.001 AEP).

- Question: What are the parameter estimates for the posterior mean frequency curve? These can be found in the Posterior Mean Summary Statistics table. Recall that the posterior mean is the average of the parameters for the 10,000 posterior distributions.

- Answer:

- Mean:

- Standard Deviation:

- Skew:

- Question: Are the precipitation frequency-based estimates reasonably consistent with the systematic, historical, and paleoflood information?

- Answer:

- Question: Was there a significant difference in the results between using one or three precipitation frequency estimates?

- Answer: