Exercise 1.25, RMC-BestFit

Effective Record Length

Disclaimer: The United States Army Corps of Engineers has granted access to the information in this hands-on exercise for instructional purposes only.

Background:

You have been tasked with estimating the effective record length for the flood hazard analysis on Example Dam. The effective record length is one of the inputs required for an RMC-RFA analysis.

The goal of an effective record length estimate is consistency between the modeled volume frequency curve uncertainty in RMC-RFA and the estimated volume frequency uncertainty in RMC-BestFit.

Required Software:

- RMC-BestFit 2.0

- Microsoft Excel

Primary Objectives:

The following tasks will be performed during this hands-on exercise:

- Estimate and calibrate the pseudo-ERL for a dataset

Additional Objectives:

If time allows complete the following additional tasks:

- Compare effective record length estimates for different datasets

Products:

At the end of this exercise, you will have an effective record length estimate that can be used in an RMC-RFA analysis.

Files Needed for Exercise:

|  | File Name | File Type | Description |
| --- | --- | --- | --- |
| 1 | E1.25.bestfit | RMC-BestFit Model | Effective Record Length project |
| 2 | E1.25_RMC Expected and Uncertainty vBETA3.xlsb | Microsoft Excel Spreadsheet | RMC Expected and Uncertainty Check Toolbox (v.Beta3) |

Task 1. Estimate the pseudo-ERL for a dataset

- Effective Record length is used to represent the width of our confidence interval and is a measure of the uncertainty in our analysis. In flow frequency analysis, a 90% confidence interval is typically used.  Because effective record length is a frequentist concept that doesn’t truly align with a Bayesian statistical framework, we can use a pseudo-effective record length (pERL) as a calibration value to ensure the RMC-RFA software is appropriately representing the uncertainty in the flow frequency curve estimated in RMC-BestFit.

- In this task you will be estimating a pERL that will appropriately represent the uncertainty estimated in the RMC-BestFit flow frequency analysis.

- Launch Microsoft Excel and open the E1.25_RMC Expected and Uncertainty vBETA3.xlsb workbook. This RMC toolbox is a draft. Check on the RMC website for the most recent version of RMC toolboxes.

- Select the Log Pearson Type 3 tab. Macros must be enabled to use the toolbox. Enable macros based on the configuration of your computer.

- Return to the RMC-BestFit software and the E1.25.bestfit project. Open the Precipitation Frequency Univariate Distribution analysis by double left-clicking the name in the Project Explorer frame.  Navigate to the Tabular Results tab in the Frequency Results tab.  If the Precipitation Frequency  Univariate Distribution analysis is already open, click on the tab at the top of the main window.

- In Step 1, enter the mean, standard deviation, and skew estimates for the posterior mode from the RMC-BestFit Precipitation Frequency Univariate Distribution analysis.

- These values can be found in RMC-BestFit in the Tabular Results tab under the Frequency Results tab. Values can be entered manually or copied and pasted from RMC-BestFit into the worksheet. When pasting into the worksheet, be sure to use the Paste Special, Match Destination Formatting (M) option to avoid changing the formatting of the worksheet cells.

- You will need to estimate and initial ERL within the Expected and Uncertainty toolbox, in Step 1 Cell F15.  This can vary depending on the input data for the analysis. For this example, the Precipitation Frequency analysis uses the paleoflood input data.  Double click on the Paleoflood Input Dataset and then click on the Summary Statistics tab. This Summary Statistics Tab gives Record Length for the Exact Data Only (Systematic) and the Nonparametric (in this case the Paleoflood Information).

- To get an initial estimate for ERL, users can take 2 approaches. 1) educated guess, or 2) users can sum up the various input data with some general guidelines. Exact Data Only Record Length can be used in its entirety, number of historic floods, 10% of the Nonparametric Record Length can be used, and 100 years for Regional Precipitation Frequency quantile priors.  Sum up these values to get an initial estimate for ERL for the Precipitation Frequency Expected and Uncertainty analysis. This can provide a very good first estimate for the ERL.  Calculate this value to compare to other ERL estimates.  For this exercise, we are going to start with a guess of 200 years for the ERL. Enter this estimate for ERL in Step 1 column F, row 15.

- Question: What value did you calculate from the input data for the initial ERL estimate?

Answer:

- Values for steps 2 and 3 have been entered for you.

- Examine the Output Parameter options in Step 2.  Row 20 allows you to enter the label for the y-axis on the plot.  Rows 21 and 22 allow you to select the min and max values for the x-axis on the plot.  Row 24 allows the user to specify the number of decimal places displayed, and row 25 allow the user to specify the number of significant figures to use when tabulating the results.  These are all user specified values that can be adjusted.

- Examine the Simulation Parameters in Step 3.  The number of realizations must be a value between 1,000 and 100,000.  The recommended default value is 1,000. The PRNG seed default is random.  This allows the toolbox to generate new, random results each time the spreadsheet is computed.  There is also an option to keep the random number generation fixed.

- In Step 4 of the Expected and Uncertainty toolbox, copy values from RMC-BestFit Precipitation Frequency Univariate Distribution analysis, including AEP, upper credible limit (95.0% CI), lower credible limit (5.0%CI), Posterior Predictive, and Posterior Mode.  These values can be found in the Tabular Results tab under the Frequency Results tab. Since we are mostly interested in moderate to extreme floods, only copy values for a range of AEPs between 1E-6 and 0.1.

- Paste the values into the worksheet table. Remember when pasting these values into the worksheet, use the Paste Special, Match Destination Formatting (M) option to avoid changing the formatting of the worksheet cells.

- In Step 5, Run the expected and uncertainty simulation by clicking the Run Simulation Button. A simulation completion percentage and post processing percentage will be reported in the bottom left of the spreadsheet toolbox.  When the analysis is complete, click the OK button.

- Step 6 displays the Output Table.  The output table contains the upper 95% confidence limit, the expected (Mean) curve, the computed (Median) curve, and the Lower 5% confidence limit that would be computed in RMC-RFA using the bootstrap method.  This is a frequentist statistical approach.

- Step 7 displays the Output Chart.  This plot shows a comparison between the curves computed using RMC-BestFit, and the curves computed using the bootstrap method which is used in RMC-RFA.  The bestfit curves are represented as open markers and the bootstrap curves are represented as solid and hashed lines.  Notice that there is a button labeled Copy Chart to Clipboard that allows you to paste this figure into a report.

- Examine the Output Chart.  Notice that the Bootstrap Upper 95% Cl is too high to match the BestFit upper 95% Cl, and too low to match the Lower BestFit 5% Cl.  Notice that the bootstrap expected curve is above the BestFit Predictive curve.  Recall that mean hazard is what we use to assess risk.  Since the Bootstrap confidence interval and expected curve does not do a satisfactory job representing the uncertainty or predictive curves in the BestFit, the ERL needs to be adjusted or calibrated.

- Now we will iterate to determine a pseudo-ERL (pERL) that will allow us to calibrate the uncertainty represented in the bootstrap analysis to the uncertainty estimated in the BestFit volume-frequency analysis.  In other words, we want to find a pERL that causes the uncertainty in BestFit and RFA to match as closely as possible.

- Now use the ERL estimate developed by summing up the values from the Input Data.  Enter this estimate for ERL in Step 1 column F, row 15.

- Click the Run Simulation button in Step 5 to re-run the simulation and select OK when the analysis is complete. Notice that the textbox on the chart displays the pERL selected and is listed along with the other parameters developed in the RMC-BestFit flow frequency analysis (mean, standard deviation, and skew).

- Question: After examining the updated chart. How does the lower 5% bootstrap confidence limit align with the lower BestFit limit when we changed our pERL to 289?

Answer:

- Question: After examining the updated chart. How does the upper 95% bootstrap confidence limit align with the upper BestFit limit when we changed our pERL to 289?

Answer:

- Question: How do the BestFit Predictive and Bootstrap Expected curves align when we changed our pERL to 289?

- Answer:

- Go back to Step 1 in the E1.25_RMC Expected and Uncertainty vBETA3.xlsb toolbox and select a new (or pseudo) effective record length in cell F15. Select 1000 for the pERL and re-run the simulation.

- If you receive a warning that says, “this simulation may take a long time, do you want to continue?”, select yes.

- Question: Examine the chart. What happens to the Bootstrap 90% CI when we changed our pERL to 1000?

Answer:

- Question: How do the BestFit Predictive and Bootstrap Expected curves align when we changed our pERL to 1000?

- Answer:

- Because frequentist and Bayesian statistical methods are not identical, it will generally be impossible to match both the upper and lower confidence limits. Because we are most interested in mean hazard and the rare flood events for dam safety, the goal of the pERL iteration is to closely match the upper 95% limit and the expected (predictive) curves.

- Continue iterating the value of pERL until you have a reasonably good match between the bootstrap expected curve and the predictive bestfit curve, and a good match between the bootstrap upper 95% CI and the BestFit Upper curve.

- Question: Report your final pERL estimate for the Precipitation Frequency flow-frequency analysis in the table below. The ERL and pERL for the other simulations have been filled in for you. This type of table would be beneficial to include in a hydrologic hazard report.

- Answer:

| - Simulation | - pERL (years) |
| --- | --- |
| - Systematic | - 80 |
| - Historic | - 135 |
| - Paleoflood | - 155 |
| - Regional Skew | - 170 |
| - Precipitation Frequency |  |

Task 2. Estimate pERL for other data inputs (if time allows)

- Conduct the same analysis as above, but on the other BestFit Univariate Distribution Analysis to compare pERL values to the table above.