Exercise 1.6, RMC-BestFit Systematic Data

Disclaimer: The United States Army Corps of Engineers has granted access to the information in this hands-on exercise for instructional purposes only.

References:

- RMC-TR-2019-07: Data Sources for Estimating Hydrologic Hazards for Semi-Quantitative Risk Assessments (https://www.rmc.usace.army.mil/Library/RMC-Publications/)

- RMC-TR-2020-03: RMC-BestFit Quick Start Guide (https://www.rmc.usace.army.mil/Library/RMC-Publications/)

- USGS Water Data for the Nation: (https://waterdata.usgs.gov/nwis)

Assignment:

You have been tasked with preparing a flood hazard analysis for Example Dam. The first step is to obtain and prepare the at-site systematic data.

It is important to collect as much at-site systematic data as practical when performing a flood hazard analysis. Common data sources can include inflow records maintained by the dam owner and gage records maintained by the USGS.

Required Software:

- RMC-BestFit

- Microsoft Excel

- HEC-DSSVue

Primary Objectives:

The following tasks will be performed during this exercise:

- Examine the at-site period of record inflow data and import time series data

- Estimate the critical inflow volume duration

- Calculate block annual maximum inflow volumes for the period of record

- Calculate a flow frequency curve using RMC-BestFit for the period of record

Additional Objectives:

If time allows complete the following additional tasks:

- Extend the period of record using available gage data

- Calculate a flow frequency curve using RMC-BestFit for the extended period of record

- Compare the frequency curves

Products:

At the end of this hands-on exercise, you will have a flow frequency curve based on the available at-site systematic data.

Files Needed for Exercise:

|  | File Name | File Type | Description |
| --- | --- | --- | --- |
| 1 | E1.6.dss | Microsoft Excel Spreadsheet | Period of Record Inflow |
| 2 | E1.6.xlsx | Microsoft Excel Spreadsheet | Period of Record Inflow |
| 3 | E1.6.bestfit | RMC-BestFit Model | Systematic Data Model |
| 4 | E1.6b.xlsx | Microsoft Excel Spreadsheet | Extended Period of Record Inflow |

Task 1.  Examine the at-site period of record inflow data and import time series data

- Background:

- The dam owner for Example Dam has maintained records of unregulated daily average inflow since construction of the dam. The period of record includes water years 1967 through 2017. The daily average inflows were estimated assuming level pool routing given the daily change in storage based on daily reservoir stage records and the daily average outflow based on gate setting records and the discharge rating curve for the outlet works.

- Open the E1.6.xlsx Period of Record spreadsheet and explore the at-site period of record inflow data.

- Question: Based on a quick visual inspection of the daily inflow plot in the Daily Inflow (chart) tab, what is the approximate date and magnitude of the largest daily average inflow?

- Answer:

- Date:

- Daily Average Inflow:

- RMC-BestFit 2.0 allows user to import time series data from different source. You will walk through three data entry method, two here and one in the addition steps. The first entry will be to pull a USGS daily gage into RMC-BestFit, the second will be to pull data from HEC-DSS, and the third is to use the manual data entry method.

- Launch the RMC-BestFit 2.0 software and open the E1.6.bestfit project using the Open Project option in the File menu.

- Create a new time series by right-clicking on Time Series Data in the Project Explorer frame. Name the new dataset 03294500 Ohio Rv at Louisville KY.

- Click the Data Entry Method dropdown menu and select USGS. This will change the Time Series Options. Now type or copy in the USGS gage number 03294500 and click Import. This brings in the daily USGS time series.

- Note the Y-axis has the label Value. Be sure to provide any time series brought in the correct Y-axis label. Under Properties find the Unit Label and rename Value to 1-Day Flow (cfs).

- Question: What is the period of record range for USGS 03294500 gage?

- Answer:

- Note the USGS gage just imported into RMC- BestFit was just for demonstration purposes to use the tool, however that time series data will not be used beyond this point.

- Create another new time series and name it Daily_Inflow. This time, select the Data Entry Method HEC-DSS. Click the DSS Filename and select the E1.6.dss, then click the DSS Pathname and select the available record. Again remember to label the Y-axis.

- Question: What is the period of record range for Daily_Inflow record?

- Answer:

Task 2.  Estimate the critical inflow volume duration

- Recall that the critical inflow volume duration should be representative of the typical duration of inflow flood hydrographs and the amount of time typically required to reach the peak reservoir stage.

- Inflow and stage hydrographs for several observed flood events are included in the E1.6.xlsx Period of Record spreadsheet. Notice that the peak reservoir stage corresponds to the intersection of the inflow and outflow hydrographs.

- Question: Based on a visual inspection, estimate the critical inflow volume duration for each of the three observed flood event plots provided in the Flood of Nov 1985 (chart), Flood of Apr 2011 (chart), and Flood of Dec 2015 (chart) tabs. Round the estimates to the nearest day.

Answer:

Critical Inflow Volume Duration for Nov 1985:

Critical Inflow Volume Duration for Apr 2011:

Critical Inflow Volume Duration for Dec 2015:

- Question: Based on your estimates for the three observed floods, estimate an overall representative critical inflow volume duration for Example Dam. Round the estimate to the nearest day. You will use this estimate in the next task to calculate annual maximum inflow volumes for the critical duration.

Answer:

Critical Inflow Volume Duration for Example Dam:

Task 3.  Estimate block annual maximum inflow volumes for the critical duration

- Recall that flow frequency analysis typically uses block annual maxima that are assumed to be independent and identically distributed (IID). It is important to check for any factors that might affect this assumption. This can include upstream regulation, land use changes, operational changes, or different flood generating mechanisms. For this hands-on exercise, assume there are no items of concern that affect the IID assumption for Example Dam.

- Recall that the block annual maxima series is typically delineated based on the water year which starts on 1 October and ends on 30 September.

- Question: Does USGS or USACE gage records always end on 30 September? Prior to calculating volume duration, what data quality check should be done to ensure proper volume duration calculation?

- Answer:

- Prior to calculating the block annual maximum critical duration, properly clean the Daily_Inflow time series record, removing data beyond 30Sept2018.

- Now calculate the daily average inflow for a duration equal to the critical inflow volume duration. This can be accomplished using the RMC-BestFit tools.

- Create a new input dataset by right-clicking on Input Data in the Project Explorer frame. Name the new dataset Period of Record.

- With the new input record, in the Properties find the Exact Data Options, click the Data Entry Method, and select Block Series. Select the Daily_Inflow for the Time Series option, Maximum from the Block Function, Water Year from the Time Block, Moving Average from the Smoothing, and type in critical duration determined in step 15 for the Period. Click Process once everything is selected.

- The result should look like the screenshot below. The values may be different depending on your estimate of the critical duration. Label the Unit Label according to the critical duration determined in step 15.

Task 4.  Calculate a flow frequency curve using RMC-BestFit for the period of record

- Create a new Bayesian analysis by selecting New Univariate Distribution Analysis from the Project menu. Name the new analysis Period of Record.

- Select the Period of Record input dataset and the Log-Pearson Type III distribution using the drop-down menus in the Properties frame.

- Run the Bayesian estimation analysis by clicking the Estimate button in the Properties frame.

- When the analysis is complete, explore the results by selecting the Frequency Plot and Tabular Results tabs.

- Congratulations. You have completed your first RMC-BestFit frequency analysis.

- Question: What are parameter estimates for the posterior mean? Navigate to the Tabular Results tab under the Frequency Results tab. The values can be found in the Posterior Mean Summary Statistics table. Recall that the posterior mode is like a computed frequency curve.

- Answer:

Mean:

Standard Deviation:

Skew:

Task 5. Extend the period of record using available gage data

- A discontinued USGS gage located just downstream of Example Dam has published daily flow data for water years 1922 through 1966. The drainage area at the gage is 1238 sq. mi. and the drainage area at Example Dam is 1186 sq. mi. The daily average flow data at the gage has already been downloaded from the USGS into a spreadsheet. Open the E1.6b.xlsx Extended Period of Record Excel workbook.

- Since the drainage areas are practically the same, a simple drainage area adjustment has been applied to the gage data to account for the difference in drainage area. The daily average flow at the gage has been adjusted to the location of Example Dam using the following equation.

Q2=Q1A2A1ϕ

QDam=QGage118612381=0.96

- Using RMC-BestFit, manual import the Daily Average Flow. Create a new time series and name it DS_USGS_Daily_Inflow.

- Edit the Time Series Options to ensure the Manual Entry and 1-Day are selected. Then modify the Start Date to 10/1/1921. Label the Unit Label 1-Day Flow (cfs). The data will fill out the tabular column and the figure.

- Copy the Daily Average Flow from E1.6b.xlsx and paste in the first value cell.

- As mentioned above the drainage area of the USGS gage and the reservoir are not the same and a drainage area ratio needs applied. Create a copy of the time series you just created and name it DS_USGS_Daily_Inflow_Area_Adjusted.

- RMC-BestFit has math functions to manipulate time series data. Click the dropdown menu next to the calculator icon, type in 0.96, and click the Multiply function.

- Create a new Input Data to calculate the critical duration for the adjusted downstream USGS gage. Name the new Input DS_USGS_Area_Adjusted_POR. Repeat step 21 to create the block maximum annual POR for this USGS gage.

- Now you have a block annual maximum input record for each of the reservoir and downstream gages. You will manual combine these two records into one. Create a new Input Data and call it Extended Period of Record.

- The default setting apply in this case, to manual enter the data. Add a row by clicking the Add Row button. From E1.6b.xlsx copy the Water Year column into the Index column. Then copy the flow values from DS_USGS_Area_Adjusted_POR Input record and paste into the Value column.

- Scroll to the bottom of the tabular data and click to Add Row. Open E1.6.xlsx if not open and copy the Water Year column into the new additional row. Then copy the flows from the Period of Record Input record in the Value column.

- The Extended Period of Record Input record should now range from 1922 to 2017.

Task 6.  Calculate a flow frequency curve using RMC-BestFit for the extended period of record

- Copy the Period of Record Univariate Distribution analysis by right-clicking on the Period of Record analysis and selecting Copy.  Name the new analysis Extended Period of Record.

- Select the Extended Period of Record input dataset using the drop-down menu in the Properties frame.

- Run the Bayesian estimation analysis by clicking on the Estimate button in the Properties frame.

- When the analysis is complete, explore the results by selecting the Frequency Plot and Tabular Results tabs under the Distribution Results tab.

- Question: What are the parameter estimates for the posterior mean? Navigate to the Tabular Results tab under the Frequency Results tab. The values can be found in the Posterior Mean Summary Statistics table. Recall that the posterior mode is like a computed frequency curve.

Answer:

Mean:

Standard Deviation:

Skew:

Task 7.  Compare the frequency curves

- Explore and compare the results for the Period of Record and Extended Period of Record univariate distribution analyses using the plot on the Distribution Results tab. Toggle back and forth between the two analyses by selecting the tab for each analysis. Or use the new RMC-BestFit comparison tool by clicking the Alternative Results and the desired analysis.

v

- Question: Did the posterior mode curve change? How much?

Answer:

- Question: Did the posterior predictive curve change?  Why?

- Answer:

- Question: Did the credible intervals change?  Why?

Answer: