Exercise 2.16, RMC-RFA

Reservoir Model and Simulation

Disclaimer: 	The United States Army Corps of Engineers has granted access to the information in this hands-on exercise for instructional purposes only.

References:

- RMC-TR-2018-03: An Inflow Volume-Based Approach to Estimating Stage-Frequency for Dams

- RMC-RFA Lecture RMC-RFA Reservoir Model

- RMC-RFA Lecture RMC-RFA Gated vs Ungated Spillways

- RMC-RFA Lecture RMC-RFA Expected Only Simulation

- RMC-RFA Software: https://www.rmc.usace.army.mil/Software/RMC-RFA/

- RMC-RFA User’s Manual

Required Software:

- RMC-RFA

- Microsoft Excel

Assignment:

You have been assigned as the Hydraulics & Hydrology (H&H) Project Delivery Team (PDT) member for a Risk Assessment for Example Dam. Your task is to enter the inputs for RMC-RFA that were developed during the RMC Base Level Issue Evaluation Study (IES). You will need to develop the reservoir routing model and perform the RMC-RFA Simulation that will develop the stage-frequency curve.

Primary Objectives:

The following major tasks will serve as an outline for the exercise:

- Explore Existing Rating Curves

- Develop Reservoir Rating Curves

- Develop RMC-RFA Reservoir Model

- Perform RMC-RFA Simulation

Additional Objectives:

If time allows complete the following task for additional learning objectives.

- Compute Overtopping Discharge

Products:

At the end of this exercise, you will have a:

- RMC-RFA model with input data, analyses, and reservoir model entered, and perform a Simulation that produces an expected stage-frequency curve.

Files Needed for Exercise:

|  | File Name | File Type | Description |
| --- | --- | --- | --- |
| 1 | E2.16.xlsx | Microsoft Excel Spreadsheet | Example Dam Rating Curves |
| 2 | E2-16.rfa.sqlite | RMC-RFA Model | Reservoir Operations Model |
| 3 | E2.16b.xlsx | Microsoft Excel Spreadsheet | Additional Tasks - Rating Curves |

Task 1.  Explore Existing Rating Curves

- BACKGROUND: It is a best practice to calibrate the reservoir model (stage-storage discharge rating curve) used in RMC-RFA to the observed stage frequency record.  In this task, we will be examining and comparing stage-discharge rating curves for a gated dam.

- Examine the Stage-Discharge plot below entitled, “Example Dam Elevation-Discharge: Gates Fully Opened”.

- This plot was developed by multiplying the full discharge capacity for a single gate by the number of gates (7).

- The orange rating curve displays the discharge values when free flow is assumed (weir flow over the spillway).

The navy blue rating curve displays the behavior of a rating curve when orifice flow is computed with the gates fully open and the water surface reaches the top of the gate openings (In this case, above elevation 1141.45-feet).

Note the inflection point in both rating curves at the top of the dam.  At this point, the computations take into account the water going over the top of the dam with weir flow.

The major take-away point for this examination is that for the same stage, if orifice flow is not accounted for, the free flow estimation greatly overestimates the flow capacity.

For example, at the top of dam elevation of 1147.45-feet, the orifice flow rating curve estimates outflow capacity at around 355,000 cfs, while the free flow rating curve estimates discharge capacity around 410,000 cfs.

It is very important to account for orifice flow appropriately when analyzing a gated spillway.

Figure 1

- Examine the Stage-Discharge plot below entitled, “Example Dam Elevation-Discharge: Best Estimate Curve”.

- In this example, the blue rating curve displays the discharge values when free flow is assumed (weir flow over the spillway).  The green rating curve displays the behavior of a rating curve when orifice flow is computed with the gates fully open and the water surface reaches the top of the gate openings (In this case, above elevation 1141.45-feet).

Note that these rating curves are identical to the curves in the previous plot above the Top of Surcharge pool.

These two “best estimate” rating curves were calibrated to the observed record in RMC-RFA.  Notice that for elevations between the Spillway and the Top of Flood Control pool, the best estimate rating curve releases much less water than the full capacity.

For example, at the Top of Flood Control at elevation 1135.45-feet, the Best Estimate curves release around 25,000 cfs, while the full capacity would have released around 250,000 cfs.

This is one of the reasons why it is so important to calibrate to the observed record rather than just using the full capacity stage-discharge rating curve in an RMC-RFA model.

Figure 2

- Question: What could be some physical reasons for the differences between this plot and the previous plot below the Top of Surcharge elevation of 1137.45-feet?

- Answer:

Task 2.  Develop Reservoir Rating Curves

- BACKGROUND:  If you are developing rating curves by hand and you must extrapolate your rating curve past the curve developed for the design pool in the Water Control Manual, it is good practice to combine the discharge from all possible sources into the Reservoir Model, including overtopping of the dam.  Do NOT simply extrapolate free-flow discharges. Instead, make sure to follow guidance in USACE EM 1110-2-1603 to develop spillway rating curves. If you are provided a rating curve, it is good standard practice to spot check the computations, checking for overtopping above the top of dam or top of gate elevation, and using orifice flow computations if the dam has gates.

- In this exercise, the individual discharge values have been developed for you in the interest of time.  You will create two reservoir Stage-Storage-Discharge rating curves; one that includes the full capacity of the outlet works, and a second rating curve that does not include outlet works.

- Open the Example Dam Rating Curves excel document called “E2.16.xlsx”.

- Examine the tabs containing stage-discharge data for the Full Capacity Spillway, Calibrated Spillway Discharge, the Embankment Overtopping (OT), Rim Dike 1 OT, Rim Dike 2 OT, Rim Dike 3 OT, and the Outlet Works.

- The Full Capacity Spillway is the navy-blue curve from Figure 1 above, and includes the full capacity of the spillway, taking orifice flow into account.

- The Calibrated Spillway discharge still accounts for orifice flow, but the discharges have been adjusted to better match the observed empirical stage data.  Calibration of reservoir models will be further explained in future lectures.

- Example Dam overtops at two locations prior to overtopping the dam, at the Rim Dike 1, and Rim Dike 2.

- Go to the “3 Combined with Outlet Works” tab.  From the preceding tabs, copy and paste the values for the Outlet Works, Full Capacity Spillway, Embankment Overtopping, Rim Dike 1, Rim Dike 2, and Rim Dike 3.  The sheet will sum the spillway and overtopping discharge and will add the outlet works full capacity discharge.

- Similarly, go to the “4 Combined w No Outlet Works” tab.  From the preceding tabs, copy and paste in the values for the Calibrated Spillway Discharge, Embankment Overtopping, Rim Dike 1, Rim Dike 2, and Rim Dike 3.  This time, we will not include the discharge from the outlet works (low flow gates).

- The rating curves computed in this task will be used for Task 3.

Task 3.  Develop RMC-RFA Reservoir Model

- Start the RMC-RFA software. Open the RMC-RFA Reservoir Operations Model “E2-16.rfa.sqlite”.

- Create a new reservoir model. Right click on the “Reservoir Models” module within the Table of Contents and select “New”.

Figure 3

- Name your Reservoir Model “Example Dam – Full Capacity Releases” and press save.

- If it’s not already open, open the Exercise 2.16 excel spreadsheet and navigate to the “Storage” tab.

- Copy and paste the stage and storage values from “Storage” tab of the excel spreadsheet into the first two columns in the RMC-RFA Reservoir Model. (Note, do not save until all reservoir data is entered.)

Figure 4

- Now paste in the combined discharge values from column N from the tab entitled “3 Combined with Outlet Works” into the “Discharge (CFS)” column in the RMC-RFA Reservoir Model.

Figure 5

- In RMC-RFA, switch to the “Reservoir Features” tab at the top of the Reservoir Model window, and enter the Top of Dam elevation, the Spillway Crest elevation, and the Inflow Design Flood peak pool elevation into this chart.  These values can be found on the “Reservoir Features” tab in your excel spreadsheet.

Figure 6

- Save your reservoir model.

- Right click on the Plot of the “Stage-vs-Storage” chart and click on the “Stage-Discharge Function” option at the bottom of the page to toggle between the two charts.

Figure 7

Figure 8

- Now, create a second reservoir model “entitled “Example Dam – No Outlet Works” using the same method as above.

- Copy and paste the Stage and Storage values from the “Storage” tab in the excel spreadsheet into the RMC-RFA reservoir model.

- This time, copy and paste the combined discharge values from Column J of the “4 Combined w No Outlet Works” tab in the excel spreadsheet into the RMC-RFA reservoir model to create a stage-storage-discharge rating curve that does not include the influence of the outlet works.

- Copy and Paste the Reservoir Features from the excel spreadsheet into the Reservoir Model and save the project.

- Examine the Stage-Discharge Plot, compare the differences from Figure 8 above.

Figure 9

- Keep the RFA model open for Task 4.  Save and close the excel spreadsheet.

Task 4.  Perform RMC-RFA Simulation

- Right click on the “Simulations” module and select “New”.

Figure 10

- Enter the name, “4-Day Example Dam_Full Capacity” in the Simulation window that opens.

- For the Input Parameters, select the following “Statistical Parameters”.

  - “4-Day Syst_Hist_RegSkew_Paleoflood_Precip” for the Volume Frequency Curve

  - “4-Day Seasonality – Stage” for the Flood Seasonality Analysis

  - “Example Dam Stage Duration” for the Reservoir Starting Stage Duration

- Select the “Example Dam – Full Capacity Releases” for the Reservoir Model.

- Check the “Simulate” boxes for the three (3) 4-day hydrograph shapes and give them each an equal weight of 1.0.

Figure 11

- Switch to the “Simulation Settings” tab and click the radio button for “Simulate Expected Frequency Curve Only”.

Figure 12

- Use “Simulate Expect Frequency Curve Only” for rapid sensitivity analysis runs. This option runs in seconds and allows you to quick produce and compare results.

- For the “Routing Options”, type in 6 for the Routing Time Window (Days), and a Routing Time Step of 1-Hour.  Keep the default Sampling Options. The routing time window should be at least as long as the inflow hydrograph shapes and should be a little longer to allow the for critical duration hydrograph to be routed.

Figure 13

- Save the project and then press the ►Compute button. RFA will now use Monte Carlo procedures to compute 10,000 simulations to create a best estimate stage-frequency hazard curve.

Figure 14

Figure 15

- Click on black arrow next to the “Empirical Frequency Curve” button at the top of the window, select Stage, and the “Example Dam Stage Frequency” empirical annual maximum stages.

Figure 16

- Question: Does the Full Capacity stage-frequency curve over- or under-estimate the observed empirical stage frequency data?

- Answer:

- Question: What physical reason could cause the curve to behave this way?

- Answer:

- Now, create a second RMC-RFA simulation and enter the name, “4-Day Example Dam_No Outlet Works”.

- Enter all the same parameters as before – except this time, use the “Example Dam – No Outlet Works” Reservoir Model. (Note, if you right click on the previous Simulation and selected copy, after entering a new name, the only change needed is the Reservoir Model selection.)

- Once you have run your simulation, click on black arrow next to the “Empirical Frequency Curve” button at the top of the window, select Stage, and the “Example Dam Stage Frequency” empirical annual maximum stages.

- Question: Does the “No Outlet Works” stage-frequency over- or under-estimate the observed empirical stage frequency data?

- Answer:

- Question: What physical reason could cause the curve to behave this way?

- Answer:

- Question: Why would we want to better match the empirical stage data?

- Answer:

- Question: What steps could we take to better match the empirical stage data?

- Answer:

Additional Objectives

Task 5.  Compute Overtopping Discharge

- BACKGROUND: In this task, you will calculate the overtopping discharge using the weir equation. Many USACE Dam Safety studies require the discharge over the dam crest to be calculated. This could be because the PMF exceeds the dam crest and/or RMC-RFA requires the “Reservoir Model” to extrapolate above the dam crest to allow the stage-frequency curve to extend beyond the dam crest.

- Open the Additional Task Rating Curves excel spreadsheet, “E2.16b.xlsx”.

- Go to the “Embankment OT” tab.  Here you will see the Pool Elevations ranging from EL 1070.45 (the invert of the outlet works) up to EL 1151.45 (The Example Dam top of dam elevation is 1147.45).

- Compute the overtopping flow over the dam using the weir equation (Q=CLH3/2).  Scroll down to the Top of Dam elevation of 1147.45.

  - C is the discharge coefficient (assume 2.6 because it is a very wide spillway),

  - L is the effective length (Provided for you),

  - H is the energy head (subtract the Top of Dam crest from the WSEL)

- Question: What are some reasons the effective length of the dam could vary?

- Answer: