Exercise 2.24, RMC-RFA

Sensitivity Analysis

Disclaimer: 	The United States Army Corps of Engineers has granted access to the information in this hands-on exercise for instructional purposes only.

References:

- RMC-TR-2018-03: An Inflow Volume-Based Approach to Estimating Stage-Frequency for Dams

- RMC-RFA Lecture Simulated Input Plots

- RMC-RFA Lecture Routed Hydrograph Plots

- RMC-RFA Lecture Sensitivity Plots

- RMC-RFA Lecture RMC-RFA Iterative Sensitivity Analysis

- RMC-RFA Software: https://www.rmc.usace.army.mil/Software/RMC-RFA/

- RMC-RFA User’s Manual

Required Software:

- RMC-RFA

- Microsoft Excel

Assignment:

RMC-RFA allows the user to perform quick sensitivity runs. This capability provides the opportunity to determine what the stage-frequency curve is most sensitive to. As there are many inputs into RMC-RFA, this can help ensure confidence in the analysis. In this exercise, participants will “turn the knobs” in RMC-RFA and determine which input parameter the stage-frequency curve is most sensitive to.

Primary Objectives:

The following major tasks will serve as an outline for the exercise:

- Reservoir Model Sensitivity

- Critical Inflow Duration Sensitivity

- Effective Record Length Sensitivity

- Flood Seasonality Sensitivity

- Reservoir Starting Stage Duration Sensitivity

- Full Uncertainty Simulation

One of the benefits of RMC-RFA is the availability to run fast iterations, by selecting the “expected only” simulation option. Using “expected only” option work through the following tasks and determine which parameters have the greatest impact on the stage-frequency curve. At the end of each task, you will pull the results into Excel and determine what impact the parameter had on the Top of Dam probability.

Classify the level of sensitivity, in comparison to the best estimate, using the following definitions:

- Primary Risk Effects: Moved Top of Dam AEP an order of magnitude or more

- Secondary Risk Effects: Moved Top of Dam AEP a half order of magnitude

- Tertiary Risk Effects: Moved Top of Dam AEP less than half order of magnitude

Additional Objectives:

If time allows complete the following task for additional learning objectives:

- Reservoir model/gate failure sensitivity

Products:

At the end of this exercise, you will have:

- An RMC-RFA model with sensitivity analyses completed and a spreadsheet comparing the results. You now have enough information to select the final RMC-RFA simulation options and run a full uncertainty simulation with a minimum of 1,000 realizations.

Files Needed for Exercise:

|  | File Name | File Type | Description |
| --- | --- | --- | --- |
| 1 | E2.24.xlsb | Microsoft Excel Spreadsheet | RMC-RFA Sensitivity Parameters |
| 2 | E2-24.rfa.sqlite | RMC-RFA Model | Sensitivity Model |
| 3 | E2.24b.xlsb | Microsoft Excel Spreadsheet | Additional Sensitivity |
| 4 | E2-24b.rfa.sqlite | RMC-RFA Model | Additional Sensitivity Model |

Task 1. Reservoir Model Sensitivity

- Exercise 2.16 Previous Sensitivity: In Exercise 2.16, you were asked to run the Full Capacity and No Outlet Works Reservoir Models. Below is the comparison between the Full Capacity and No Outlet Works RMC-RFA simulations from Exercise 2.16.

- Question: How sensitive is the stage-frequency curve to the Reservoir Model at Top of Dam?

Circle an Answer: 	a.  Primary: Moved Top of Dam AEP an order of magnitude or more

b.  Secondary: Moved Top of Dam AEP a half order of magnitude

c.  Tertiary: Moved Top of Dam AEP less than half order of magnitude

- Question: If the hydrologic hazard curve is not sensitive to the Reservoir Model at the Top of Dam, does it matter which Reservoir Model is selected for the stage-frequency curve? Why?

- Answer:

Task 2. Critical Inflow Sensitivity

- BACKGROUND: It is common to test more than one critical duration to ensure the selected critical duration is representative of the observed floods in the watershed. A 5-day critical inflow duration will be examined for comparison.

- Open RMC-RFA and load RMC-RFA Sensitivity Model “E2-24.rfa.sqlite”.  Click the drop-down arrow for each of the parameters in the project explorer. Notice a 5-day inflow hydrograph, a 5-day volume frequency curve, and a 5-Day Flood Seasonality have been added to the RMC-RFA model.

- Create a new Simulation and name it 5-Day_No Outlet Works.

- For the Input Parameters, select the following “Statistical Parameters”.

  - “5-Day Syst_Hist_RegSkew_Paleo_Precip” for the Volume Frequency Curve

  - “5-Day Flood Seasonality” for the Flood Seasonality Analysis

  - “Example Dam Stage Duration” for the Reservoir Starting Stage Duration

- Select the “Example Dam – No Outlet Works” for the Reservoir Model.

- For the Inflow Hydrographs, check the “Simulate” box for the one 5-day hydrograph shape and give it a weight of 1.0. Normally, 3-5 hydrograph shapes are preferred, but for the sake of time, we’ll just use one hydrograph for this exercise.

- Switch to the “Simulation Settings” tab and click the radio button for “Simulate Expected Frequency Curve Only”.

- Use “Simulate Expect Frequency Curve Only” for rapid sensitivity analysis runs. This option runs in seconds and allows you to quick produce and compare results.

- For the “Routing Options”, type in 7 for the Routing Time Window (Days), and a Routing Time Step of 1-Hour.  Keep the default Sampling Options.

- Save the project and then press the ►Simulate button. RFA will now use Monte Carlo procedures to compute 10,000 realizations to create a best estimate stage-frequency hazard curve.

- Open the Sensitivities spreadsheet “E2.24.xlsb”. This spreadsheet allows a direct comparison between the RMC-RFA simulations. Note, the Reservoir Model sensitivity simulations are already entered and showing on the “Stage-Frequency Plot” tab.

- Within RMC-RFA, click the Get Tabular Output button. This opens a table with the stage-frequency curve tabulated. Copy and paste the 5-day simulation results (AEP and Expected Curve) to the “Data Input” tab into column AE and AF in the spreadsheet.

- When coping and pasting from RMC-RMA, always copy and paste both the AEP and frequency curve columns. The output AEP values can vary from one simulation to another.

- A trick to copying from RFA – select the first cell in a column, hold shift, then use the scroll bar to reach the last data entry, click in the last cell to select the data and control “C” to copy the data.  If you would like to select the data in the entire table, use control “A” to select all, then control “C” to copy the data.

- In the spreadsheet “Stage-Frequency Plot” tab, turn off and/or on the curves that are getting compared. Click on the Chart and some options will appear to in the upper right corner. Click on the Chart Filters button.

- Click on and/or off the time series that are getting compared. Then click “Apply” at the bottom. At this point, the 4 Day No Outlet Works and 5 Day No Outlet Works should be checked.

- Question: How sensitive is the stage-frequency curve to the critical duration at Top of Dam?

Circle an Answer: 	a.  Primary: Moved Top of Dam AEP an order of magnitude or more

b.  Secondary: Moved Top of Dam AEP a half order of magnitude

c.  Tertiary: Moved Top of Dam AEP less than half order of magnitude

Task 3. Record Length Sensitivity

- BACKGROUND: This task will look at a common scenario for USACE Dam Safety studies. Often stage-frequency curves will reflect a volume-frequency analysis with systematic and historical data. Most gages in the U.S have less than 100 years of record, and dams tend to have 60 years or less. Therefore, often the effective record length (ERL) is between 50 and 100 years for the systematic and historic data.

- This task will look at just changing the RMC-RFA Volume Frequency Curve ERL for the 4-Day systematic and historical data.

- Create a new “Volume Frequency Curve” and name it 4-Day Syst. Enter the following parameters, save, and press the ►Compute button, and Save.

- This volume frequency information was developed using only systematic information.  Note the systematic ERL is 80. Recall that the original volume-frequency curve includes historic information, regional skew, paleoflood analysis and precipitation-frequency rainfall runoff results and had an estimated ERL of 350 years.  Testing the volume-frequency curve will show the effect adding regional information had on the stage-frequency curve.

- Create a copy of the 4-Day Syst and name the copy 4-Day Syst_Hist_ERL150. Change the ERL to 150, save, and press the ►Compute button, and Save. You may need to double click in the project explorer window to ensure the new volume-frequency curve window has opened before you make modifications.

- When paleoflood information is added to the volume-frequency curve along with regional information, it is possible to see the ERL increase much more. Test how a 500 ERL would impact the stage-frequency curve.

- Create a copy of the 4-Day Syst_Hist_ERL150 and name the copy 4-Day Syst_Hist_ERL500. Change the ERL to 500, save, and press the ►Compute button, and Save.

- Now create three new simulations using the three new volume frequency curves for the 4-day systematic and historical data.

- Create a new “Simulation” and name it 4-Day_No Outlet Works_Syst. Enter the following parameters, reservoir model, and check the three 4-day inflow hydrographs with 1.0 weight. (You will need to un-check the 5-day hydrograph shape.)

- Under the “Simulation Settings” tab, select the expected curve only option, enter a 6-day time window, and leave the remaining options as default. Save the model.

- Now click the ►Simulate button. Once the simulation is done, transfer the results to the spreadsheet.

- Create two copies from the “4-Day_No Outlet Works_Syst” and name them 4-Day_No Outlet Works_Syst_Hist_ERL150 and 4-Day_No Outlet Works_Syst_Hist_ERL500.

- Change the “Volume Frequency Curve” in each new simulation to the appropriate ERL. Save and click the ►Simulate button for each. Once again transfer the results to the spreadsheet.

- On the spreadsheet “Stage-Frequency Plot” tab, select only the three 4-day syst_hist_ERL curves to view the change due to increased ERL.

- Question: How sensitive is the stage-frequency curve from the Syst_Hist with ERL 80 to ERL 150 at Top of Dam?

Circle an Answer: 	a.  Primary: Moved Top of Dam AEP an order of magnitude or more

b.  Secondary: Moved Top of Dam AEP a half order of magnitude

c.  Tertiary: Moved Top of Dam AEP less than half order of magnitude

- Question: How sensitive is the stage-frequency curve from the Syst_Hist with ERL 80 to ERL 500 at Top of Dam?

Circle an Answer: 	a.  Primary: Moved Top of Dam AEP an order of magnitude or more

b.  Secondary: Moved Top of Dam AEP a half order of magnitude

c.  Tertiary: Moved Top of Dam AEP less than half order of magnitude

- The ERL sensitivity above is based on just changing the ERL for the volume frequency curve with systematic and historical data only. However, during our RMC-BestFit Exercises, we developed a range of volume frequency curves from the systematic and historical to the final curve with paleoflood, regional skew, and rainfall runoff results included. Look now at how the increased information in the volume frequency curves and the changing ERL affect the stage-frequency curve.

- We already created a volume frequency curve with the final RMC-BestFit parameters that includes the extended systematic, historic, regional skew, paleoflood, and precipitation-frequency rainfall runoff information called 4-Day Syst_Hist_RegSkew_Paleoflood_Precip.

- In the spreadsheet on the “Stage-Frequency Plot” tab, select the following time series to compare: 4-Day No Outlet Works, 80 Year ERL (4-Day No Outlet Works_Syst), and 4-Day Syst_Hist_ERL500.

- Question: The ERL increased from 80 to 500 when we estimated the effects of adding paleoflood information.  When we examined the actual final 4-day volume frequency parameters from our RMC-BestFit analysis, the estimated ERL was 350. Why are the 4-Day No Outlet Works and 4-Day_No Outlet Works_Syst_Hist_ERL500 stage-frequency curves nearly identical?

- Answer:

- Question: How sensitive is the stage-frequency curve at the Top of Dam when all the information is added, including regional information and paleoflood?

Circle an Answer: 	a.  Primary: Moved Top of Dam AEP an order of magnitude or more

b.  Secondary: Moved Top of Dam AEP a half order of magnitude

c.  Tertiary: Moved Top of Dam AEP less than half order of magnitude

Task 4. Flood Seasonality Sensitivity

- In Exercise 2.8, two “Flood Seasonality” analyses were done. However, a sensitivity was not run to determine the effects. The two “Flood Seasonality” analyses are included in the RMC-RFA model.

- Create a new “Simulation” by creating a copy of the “4-Day_No Outlet Works” and naming it 4-Day_No Outlet Works_Flood Seasonality.

- Modify the “Flood Seasonality” selection for the simulation by selecting the Flood Seasonality analysis. Click the ►Simulate button, save the model, and transfer the results to the spreadsheet.

- In the spreadsheet on the “Stage-Frequency Plot” tab, select the following time series to compare: 4-Day No Outlet Works, and 4-Day No Outlet Works_Flood Seasonality.

- Question: How sensitive is the stage-frequency curve to the Flood Seasonality at Top of Dam?

Circle an Answer: 	a.  Primary: Moved Top of Dam AEP an order of magnitude or more

b.  Secondary: Moved Top of Dam AEP a half order of magnitude

c.  Tertiary: Moved Top of Dam AEP less than half order of magnitude

Task 5. Full Uncertainty Simulation

- Throughout our sensitivity analysis, we have performed “expected only” simulations because they run very quickly and make it easy to compare differences in expected stage-frequency curves.

- Now we will run our final Full Uncertainty Simulation for the project. When running the final stage-frequency curve simulation, it is important to select a minimum of 1,000 realizations to obtain a reasonably accurate estimate of the 90% confidence interval. Full Uncertainty simulations take longer to run as you increase the number of realizations.

- Make a copy of the 4-Day Example Dam_No Outlet Works simulation. Name the new simulation 4-Day_No Outlet Works_Full Uncertainty.  Close all windows to ensure you are working in the correct window.  Re-open the Full Uncertainty simulation you just created.

- The simulation parameters will remain the same, but the Simulation Settings will change.  Toggle on the “Simulate Full Uncertainty” Simulation type, select 1,000 for the Number of Realizations, and make sure the “Skip Inflow Events with AEP>” is set to 0.99.

- Click the ►Simulate button, save the model when complete.  Because we have run the full uncertainty run, all four curves will be computed: the expected curve, the 90% confidence interval (95% and 5% Confidence Limits), and the median curve.

- Running a full uncertainty simulation will have a longer run time. In the “Simulation Complete!” window below, we can see the full uncertainty run will take longer, in this example around 12.5 minutes, as compared to the few seconds it took to run the expected only simulations.

- Click on the Tabular Output button to examine the AEP and 4 stage-frequency curves that have been computed. To select the data, right click in the table and click “Select All”, then right click and select “Copy”.  (We don’t need to copy the table headers this time because the table headers are already present in the spreadsheet).

- Paste your results into Column P through Column T of your spreadsheet.

- Go to the Chart Setup tab and Change the Chart Title in cell B2 to “Example Dam Adopted Stage-Frequency Curve”.  This will change the title on the plot tab.

- In the spreadsheet on the “Stage-Frequency Plot” tab, select the following time series to include in your plot: Upper, Lower, 4-Day Expected Stage-Frequency Curve, and 4-Day Median Stage-Frequency Curve.

- You have now created a plot that could easily be included into a report or a presentation.  Consider adding a text box with the AEP of critical elevations, such as top of dam and PMF.

- On the Data Input tab, go to cells B25/B26 through D25/D26. The spreadsheet has an interpolate function that allows computation of AEP at key elevations.  The elevations and titles of the key elevations can be changed on the Chart Setup tab if desired. For this example, the Spillway Crest, Top of Dam and PMF elevations were entered.

- Question:

- What is the frequency and return interval for the Spillway Crest?

- What is the frequency and return interval for the Top of Dam?

- What is the frequency and return interval for the PMF?

- (Remember to examine the expected curve to answer these questions when in a risk context.)

Answer:

|  | Spillway Crest | Top of Dam | Base Level IES PMF |
| --- | --- | --- | --- |
|  | Elevation (ft, NAVD88) | Elevation (ft, NAVD88) | Elevation (ft, NAVD88) |
|  | 1,097.45 | 1,147.45 | 1,149.45 |
| AEP |  |  |  |
| Return Interval |  |  |  |

Additional Objectives

Task 6. Reservoir model/gate failure sensitivity

- As discussed in many lectures throughout this exercise, the stage-frequency curve can be quite sensitive to the reservoir model and should be calibrated to match the empirical frequency. Another aspect of reservoir modeling that can inform risk assessments is the change in the stage-frequency curve when considering gate failure. In this task we will determine the model sensitivity to the reservoir model using a gate failure scenario.

- Gate Reliability sensitivities are often run for a minimum of two scenarios; half of the spillway gates have failed, and all the spillway gates have failed.  This provides a range of risk impacts for the full range of possibilities from all the gates functioning properly to none of the gates operating properly. It’s important to bracket the problem as much as possible when working with risk assessments for dams.

- Open the RMC-RFA model “E2-24b.rfa.sqlite”. Also open the spreadsheet “E2.24b.xlsb”.

- In the spreadsheet, examine the “Full Capacity – No Outlet Works” tab.  Column B contains the maximum spillway releases for the dam, when using all 7 Tainter Gates.

- Column M contains the full capacity releases including the spillway releases and overtopping for the main embankment and 3 rim dikes. Outlet works are not included.

- In RMC-RFA, make a copy of the “Example Dam – Full Capacity Releases” reservoir model and name the copy Example Dam – Max Releases_7Gates.

- Copy the 7-Gates Total Discharge rating curve from Column M and paste into the new reservoir model in RMC-RFA and save the model.

- Right click within the plot and select “Stage-Discharge Function”. The stage-discharge curve for 7 gates maximum releases will be displayed as seen in the image below.

- Go back into the spreadsheet and examine the tab “Gate Reliability - 3 Gates”. This spillway rating curve shown in columns G and H represent the conditions if 4 of the 7 spillway gates on this dam were to fail and only 3 spillway gates were operational. The combined rating curve including spillway releases for 3-gates and overtopping is shown in columns R and S.

- Copy the Example Dam – Max Releases_7Gates reservoir model and name the copy Example Dam – Max Releases_3Gates.

- Copy and paste the 3-Gates Total Discharge rating curve from Column S into the new reservoir model in RMC-RFA and save the model.

- Right click within the plot and select “Stage-Discharge Function”. The stage-discharge curve for maximum releases will display as seen in the image below.

- Create two new simulations.  Copy the expected only “4-Day Example Dam_No Outlet Works” Simulation and name the new simulations “Max Releases 7 Gates” and “Gate Reliability 3 Gates” respectively.

- Open the new simulations and select the corresponding reservoir model (Max Releases_7Gates and Max Releases_3Gates)

- Run the simulations and copy out the tabular output into the “Data Input” tab in the Additional Sensitivities spreadsheet and compare the results.

- Compare the results from all spillway gates functioning with half of the gates not functioning (loss of 4 gates).  Select the “Max Releases 7 Gates” stage-frequency curve with the “Gate Reliability 3 Gates” stage-frequency curve.

- Question: How sensitive was the Top of Dam AEP to gate reliability (loss of 4 gates)?

Circle an Answer: 	a.  Primary: Moved Top of Dam AEP an order of magnitude or more

b.  Secondary: Moved Top of Dam AEP a half order of magnitude

c.  Tertiary: Moved Top of Dam AEP less than half order of magnitude

- Make a copy of the “Example Dam – Max Releases_3Gates” Reservoir Model and name the copy “Example Dam – Max Releases_NoGates”

- Paste in the spillway rating curve from the “Gate Reliability – No Gates” tab in the spreadsheet from Column K.  Notice that there is no spillway gate flow, only overtopping discharge included.

- Make a copy of the “Gate Reliability – 3 Gates” simulation and name the new simulation “Gate Reliability - NoGates”.

- Select the appropriate reservoir model (Example Dam – Max Releases_NoGates)

- Run the simulation and copy out the tabular output into the spreadsheet.

- Compare the results from all spillway gates functioning with all gates not functioning (loss of 7 gates).  Select the “Max Releases 7 Gates” stage-frequency curve with the “Gate Reliability No Gates” stage-frequency curve.

- Question: How sensitive was the Top of Dam AEP to gate reliability (loss of all 7 gates)?

Circle an Answer: 	a.  Primary: Moved Top of Dam AEP an order of magnitude or more

b.  Secondary: Moved Top of Dam AEP a half order of magnitude

c.  Tertiary: Moved Top of Dam AEP less than half order of magnitude