Exercise 2.3, RMC-RFA Input Data

Disclaimer: 	The United States Army Corps of Engineers has granted access to the information in this hands-on exercise for instructional purposes only.

References:

- RMC-TR-2018-03: An Inflow Volume-Based Approach to Estimating Stage-Frequency for Dams

- RMC-RFA Lecture RMC-RFA Input Data

- RMC-RFA Software: https://www.rmc.usace.army.mil/Software/RMC-RFA/

- RMC-RFA User’s Manual

Required Software:

- RMC-RFA

- Microsoft Excel

Assignment:

You have been assigned as the Hydraulics & Hydrology (H&H) Project Delivery Team (PDT) member for a USACE Risk Assessment for Example Dam. Your task is to enter the inputs for RMC-RFA that were developed during the RMC Base Level Issue Evaluation Study (IES).

You will enter the following data into RMC-RFA software: systematic inflow record, the appropriate inflow hydrographs that fit the critical inflow duration, the systematic reservoir elevation record, and RMC-BestFit inflow volume frequency Log Pearson Type III (LPIII) parameters and Effective Record Length (ERL).

Primary Objectives:

The following major tasks will be performed during this exercise:

- Enter RMC-RFA Input Data – Systematic Inflows

- Enter RMC-RFA Input Data – Inflow Hydrographs

- Enter RMC-RFA Input Data – Stage Gage

- Enter RMC-RFA Input Data – Volume Frequency Curve

Products:

At the end of this hands-on exercise, you will have an:

- RMC-RFA model with a systematic inflow record, three inflow hydrographs, a systematic stage record, and an RMC-BestFit inflow volume frequency curve.

Files Needed for Exercise:

|  | File Name | File Type | Description |
| --- | --- | --- | --- |
| 1 | E.2.3.xlsx | Microsoft Excel Spreadsheet | RMC-RFA Input Parameters |

Task 1.  Enter RMC-RFA Input Data – Systematic Inflows

- Background:

Example Dam is a high hazard dam located in Yourhome Towne, USA on the Example River and regulates 1,186 square miles of drainage area. Example Dam has a gated concrete gravity structure spillway combined with an earthen embankment dam with a hydroelectric generating station and 3 saddle dikes.  There are seven (7) 40’ x 37’ tainter gates on the spillway and a low flow outlet including two 5.6’ x 10’ Bonnet Gates.

- For a Periodic Assessment (PA) or Base Level IES, the desired input data files for an RFA model would normally include 1 period of record inflow dataset, 3-5 inflow hydrograph shapes, 1 period of record stage gage data set, and Volume Frequency curves for your critical duration and any other durations you are studying.

- Double click on the RMC-RFA icon to start the RMC-RFA software.

Figure 1

- Go to File and click on “New Project” to create a new RFA project. Provide a name, E2-3 and a Project Location (file directory).

Figure 2

- Expand all the arrows in the Input Data tab to see the four sources of input data for an RMC-RFA simulation.

Figure 3

- Open the RMC-RFA Input Parameters excel spreadsheet entitled, E2.3.xlsx.

- In RMC-RFA, right click on “Discharge Gage” in the project explorer and click “New”.

Figure 4

- Copy the name and description from the Discharge Data tab in the excel spreadsheet and paste it into the RFA Discharge project window.

- Next, enter the Start and End dates.  You can either click directly on the numbers in the cell and type a number to change the values, or you can use the calendar pull down to navigate to the dates.

Figure 5

- Click Resize Table. The table will automatically resize to the date range you entered.

Figure 6

- Copy and Paste the inflow discharge values from the excel spreadsheet into the RFA model under the Flow (CFS) column.

Figure 7

- A plot of the inflow discharge versus time will automatically appear.

Figure 8

- Save your simulation and close the Discharge Gage window.

Figure 9

Task 2.  Enter RMC-RFA Input Data – Inflow Hydrograph

- Inflow hydrographs should represent the largest floods on record with the shortest timestep available. In most cases, this will be 1-hour timesteps. The length of the inflow hydrograph record should also be appropriate for the critical duration. The inflow hydrograph has to be at least as long as the critical duration but should not be much longer. Often inflow hydrographs are pulled from the events used to determine the critical duration for the RMC-BestFit inflow volume frequency curve.

- If it is not already open, open the RFA Input Parameters excel spreadsheet entitled, E2.3.xlsx.

- In RMC-RFA, right click on Inflow Hydrograph in the project explorer and click New.

Figure 10

- Using the data on the Inflow Hydrograph tab in the excel spreadsheet, fill in the data for the April 2008 inflow hydrograph. Remember that you need to click on “Resize table” after you have adjusted the name, description, start date and time, end date and time, and time step.

Figure 11

- Save your new inflow hydrograph and close the window.  You will notice that your new input data has appeared in the RFA project explorer.

Figure 12

- Repeat the steps above to enter the December 2015 and the April 2017 flood events. Once you have completed the steps, all three inflow hydrographs will appear in the RMC-RFA project explorer.

Figure 13

Task 3.  Enter RMC-RFA Input Data – Stage Gage

- RMC-RFA will use the stage gage date entered later for the Empirical Frequency Curve. In most cases the stage gage data is entered as daily timestep.

Figure 14

- Following the steps outlined above, create a new “Stage Gage”. Use the information in the excel spreadsheet under Pool Elevation to populate the new Stage Gage data set.

- Question: What time range did you enter for the Stage Gage?

- Answer:

Task 4.  Enter RMC-RFA Input Data – Volume Frequency Curve

- Volume frequency curves can come from multiple sources, however for USACE Dam and Levee Safety risk assessments, it is recommended that the RMC-BestFit is used to develop the volume frequency curve. With the current version of RMC-RFA and standard practices, the LPIII distribution will be the primary distribution for the inflow volume-frequency curve.

- If not already open, open the RFA Input Parameters excel spreadsheet entitled, E2.3.xlsx.

- In RMC-RFA, right click on “Volume Frequency Curve” in the project explorer and click “New”.

Figure 15

- Using the data on the “Volume Frequency Curve” tab in the excel spreadsheet, fill in the data for the new volume frequency curve. Once the data is entered, click the ►Compute button.

- Question: What Probability Distribution did you select?

- Answer:

- Question: Fill out the table below with the RMC-RFA volume frequency analysis results. How do the RMC-RFA results compare to the RMC-BestFit volume frequency result?

- Answer:

|  | - Probability | - Upper (0.95) | - Posterior Mean /Median (0.50) | - Lower (0.05) |
| --- | --- | --- | --- | --- |
| - RMC-BestFit | - 1E-06 | 425,741 | 252,312 | 158,728 |
| - RMC-RFA |  |  |  |  |
| - RMC-BestFit | - 1E-04 | 202,943 | 147,839 | 110,511 |
| - RMC-RFA |  |  |  |  |
| - RMC-BestFit | - 1E-02 | 79,230 | 68,560 | - 59,415 |
| - RMC-RFA |  |  |  |  |

- Question: What impact, if any, in this case would the differences between RMC-RFA and RMC-BestFit have on the loading curve (stage-frequency)?

- Answer:

Figure 16