Exercise 2.8, RMC-RFA Analyses

Disclaimer: 	The United States Army Corps of Engineers has granted access to the information in this hands-on exercise for instructional purposes only.

References:

- RMC-TR-2018-03: An Inflow Volume-Based Approach to Estimating Stage-Frequency for Dams

- RMC-RFA Lecture RMC-RFA Flood Seasonality

- RMC-RFA Lecture RMC-RFA Reservoir Starting Stage Duration and Empirical Frequency Curve

- RMC-RFA Software: https://www.rmc.usace.army.mil/Software/RMC-RFA/

- RMC-RFA User’s Manual

Required Software:

- RMC-RFA

- Microsoft Excel

Assignment:

You have been assigned as the Hydraulics & Hydrology (H&H) Project Delivery Team (PDT) member for a Risk Assessment for Example Dam. Your task is to enter the inputs for RMC-RFA that were developed during the RMC Base Level Issue Evaluation Study (IES).  You will perform the RMC-RFA analyses required for an RMC-RFA Simulation: Flood Seasonality, Reservoir Starting Stage Duration, and Empirical Frequency Curve.

Primary Objectives:

The following major tasks will serve as an outline for the exercise:

- Perform RMC-RFA Flood Seasonality Analysis

- Perform RMC-RFA Reservoir Starting Stage Duration Analysis

- Perform RMC-RFA Empirical Frequency Curve Analysis

Additional Objectives:

If time allows complete the following task for additional learning objectives.

- Additional Flood Seasonality Analysis

Products:

At the end of this exercise, you will have a:

- RMC-RFA model with a flood seasonality, reservoir starting stage duration, and empirical frequency curve analyses complete for use in an RMC-RFA Simulation.

Files Needed for Exercise:

|  | File Name | File Type | Description |
| --- | --- | --- | --- |
| 1 | E2-8.rfa.sqlite | RMC-RFA Model | RMC-RFA Analyses Model |

Task 1.  Perform RMC-RFA Flood Seasonality Analysis

- BACKGROUND:   Example Dam is a high hazard dam located in Yourhome Towne, USA on the Example River and regulates 1,186 square miles of drainage area. Example Dam has a gated concrete gravity structure spillway combined with an earthen embankment dam with a hydroelectric generating station and 3 saddle dikes.  There are seven (7) 40’ x 37’ tainter gates on the spillway and a low flow outlet including two 5.6’ x 10’ Bonnet Gates.

- Now we will use the input data you created in the previous exercise to perform the various analyses required for RMC-RFA simulation.

- For a Periodic Assessment (PA) or Base Level IES, the desired analyses performed for an RFA model would normally include 1 flood seasonality, 1 reservoir starting stage duration, and 1 empirical frequency curve.

- Open RMC-RFA software and open the RMC-RFA project entitled E2-8.rfa.sqlite.

- Create a new Flood Seasonality analysis. This can be accessed through the top dropdown menu as seen in the figure or by right clicking on “Flood Seasonality”. Modify the name to 4-Day Flood Seasonality.

- Tip: You will need one Seasonality Analysis per each Volume Duration Frequency (VDF) curve associated with an inflow volume duration in your RMC-RFA model. If you are looking at multiple critical volume durations, then you will have one Seasonality Analysis for each volume duration.

- Parameter Overview:

  - Threshold Flow (cfs):

- Begin by selecting a flow that corresponds to a specific frequency level, (i.e., 5-year or 10-year).  This flow should be high enough so that only larger events are considered in the analysis, and the data set needs to be large enough to reduce uncertainty from sampling error with a sufficiently large sample.  Aim for a sample sizes of 30-40 floods.

  - Critical Duration (days):

- The duration should correspond to the VDF curve duration.

  - Max Events per Year:

- A partial duration technique is used to expand the sample size, allowing more than one flood event per year.  Caution should be used to ensure hydrologically independent events.

  - Min Days Between Events:

- Helps to identify independent events.  Ex: if the Critical Duration is 2-days, a minimum of 7-days between events may be sufficient to ensure independence.  Use the inflow time series to visually inspect for the typical time between large flow events.

- Fill out the Parameters for the new Seasonality Analysis. Use the data presented below.

- Choose a threshold flood. Press ►Compute to run the analysis. Start with the 5-year RMC-BestFit Posterior Predictive value and modify according to the target 30-40 floods. Below is from the RMC-BestFit “Precipitation Frequency Sensitivity” Bayesian analysis.

| - Return Year | - AEP | - Posterior Predictive |
| --- | --- | --- |
| - 10 | - 0.1 | - 37,060 |
| - 5 | - 0.2 | - 28,386 |
| - 3.33 | - 0.3 | - 23,344 |
| - 2 | - 0.5 | - 16,826 |

- Question: What threshold flood did you use? How many floods did it create?

- Answer:

Task 2.  Perform RMC-RFA Reservoir Starting Stage Duration Analysis

- The “Reservoir Starting Stage Duration” curves represent the percent of time during which antecedent reservoir stages are exceeded. The “Flood Seasonality” and “Reservoir Starting Stage Duration” curves work together to select the starting reservoir elevation for an RMC-RFA simulation.

- Create a new “Reservoir Starting Stage Duration” Analysis. Give it a new name, Example Dam Stage Duration.

- From the Parameters section of the Analysis tab, use the pull-down tab to Select Stage Gage.  For this analysis, there is only one discharge gage, and it includes the extended dataset developed in an RMC-BestFit exercise previously in the course.

- Parameter Overview:

  - Pool Change Threshold:

- The threshold attempts to capture changes in stages prior to any given flood event. To estimate the pool change threshold, it can be useful to examine rates of rise on the rising limb of observed hydrographs.  The goal is to remove stages that occur during the peak of a flood hydrograph.

  - Typical High Pool Duration:

- Similar to pool change threshold, the high pool duration is used to filter out large events from an RMC-RFA Simulation starting pool elevation. To estimate the typical high pool duration, it can be useful to examine the reservoir stage period or record and examine how long the pool typically stays up after a large inflow event.

- Begin by examining the period of record “Stage Gage” data by double clicking on the dataset in the Project Explorer window in RMC-RFA.

- To zoom in on the plot, hover over the plot and left click.  Select Zoom in the pop-up window, and the mouse will change into a magnifying glass symbol.

- You can zoom in on individual flood events within the period of record reservoir dataset to examine more closely.

- To return to the full plot, right click while hovering the mouse over the plot, and select Zoom Full Extent.

- To select a “Pool Change Threshold”, you need to select several years with smaller flood events to observe a maximum stage rate of rise per day.

- To select a “Typical High Pool Duration”, you should again look at the stage record in RMC-RFA, evaluate several flood events to get an average duration of the larger flood events. Recall that you are estimating the duration of time that the reservoir peak pool stays up before dropping back down to a normal pool level in days.

- Question: Roughly guessing at the duration based on the Example stage gage in RMC-RFA, what is your guess at the duration?

- Answer:

- Enter the values below into RMC-RFA and press the ►Compute button.

- The updated plot shows blue for the observed Stage Data and red for the newly selected Starting Stage Data.  The stages included in the Starting Stage Data will be used in RMC-RFA as the antecedent reservoir pools in the simulations.

- Notice that the large peaks have been eliminated from potential starting stages, which is more typical of antecedent flood conditions. In other words, we make the assumption that it would be unlikely for a very large flood event to begin at the peak of a previous large flood event. For this dataset, there are very long durations for high pools, but the peaks are removed.

- Click on the “Duration Curves” tab to view the plot. This provides a visualization of the stage-duration curves an RMC-RFA Simulation would select starting pool elevations from.

- Save your analysis by clicking on the diskette symbol at the top of the project window.

Task 3.  Perform RMC-RFA Empirical Frequency Curve Analysis

- “Empirical Frequency Curve Analysis” is used to develop the annual maximum stage time series that you will use in an RMC-RFA Simulation. The observed data will later be used to verify the RMC-RFA Simulation results.

- Create a new “Empirical Frequency Curve”.  Provide a new name, Example Dam Stage Frequency, and description.

- Fill out the remaining parameters in RMC-RFA according to the figure below. Then press the ►Compute button.

- For most USACE Dam Safety risk assessment studies, Weibull plotting position and water year will used as the default settings. The duration will almost always be 1-day, because the stage gage data is usually daily data. After computing, RMC-RFA reports the annual maximum stage frequency curve.

Task 4.  Additional Flood Seasonality Analysis

- At times the flood seasonality will need adjusting from using the standard flow threshold. In some cases, there may be additional information based on stages that better represents the frequency of peak pools by season.

- Create a new Seasonality analysis in RMC-RFA and name it “4-Day Seasonality - Stage”.

- To manually edit the table in RMC-RFA, right click in the Tabular Results portion of the Seasonality window and select “Allow Manual Data Entry”.

- Enter the frequency values below into the Seasonality Analysis and Save the analysis by pressing the save button in the menu strip.  These values were developed manually by analyzing the frequency of peak pools by season.

- Question: Look back at the original seasonality using flow thresholds. What is a major difference between that flood seasonality and this one based on an outside study of stages?

- Answer: